# lyra-cpp — Session Log

Running EOD log. Newest entry on top. Short rough-outline format.

## 2026-06-30 — RELEASED v0.8.0 (companion-app launch + CW external/serial keyer + the post-v0.7.0 batch)

Cut v0.8.0 bundling everything on main since v0.7.0 + today's work.

### Today's work (this session)

**#193 Companion-app launch on profile pick — 40afa46, operator-confirmed

("bingo") with VarAC + MSHV. New `ProfileManager::loadByUser()`/`userLoaded` distinguish an explicit pick from the silent auto-recall; new `src/profile/CompanionLauncher.{h,cpp}` (Enable / name / command+args / start-delay; `startDetached`, .bat via `cmd.exe /c`; already-running skip; no auto-kill). Per-machine QSettings profileLaunch/*, filtered by isMachineSpecificKey → never in the portable profile/.lyra.** Fires on the front ProfilePanel combo (loadByUser) + the Settings Load button; the auto-recall combos still call load(). Profiles tab reworked (companion group + width-capped 2-col auto-recall grid).

#159 DSP Options — CLOSED-AS-SCOPED. Filter-type-only tab is at parity for

what modern-HW/Qt6 can trade; added a one-line note cross-referencing Audio→VAC as the one tunable latency. Rationale in memory.

#186 CW keyer settings — CLOSED-AS-AT-PARITY (no code). Diffed the actual

Thetis CW screen + the _cw wire struct: every gateware-honorable knob is already in Settings→CW; the two not-present items (Auto Mode Switch, Software sidetone) are host-only with no wire field → inert on HL2, deliberately not ported.

#171 External CW keyer / Winkeyer — BUILT (default-OFF, bench pending).

Gateware verify (HL2+ ak4951v4 CW=2) settled it: KEY-jack keys the carrier autonomously (= the shipped CW-2 path). Piece A = WKmini→HL2 jack, Iambic OFF (a Settings→CW note). Piece B = new src/cat/SerialCwKey.{h,cpp} (sibling of SerialPtt): polls a COM port CTS/DSR pin → HL2Stream:: requestCwKeyFromSerial() → the #105 cwx/cwx_ptt path, holds cwx_ptt across the break-in hang; NO host iambic. Settings→CW "CW key over COM port" group, default-OFF, per-machine QSettings. Ported Lyra-native from Thetis Console/cwkeyer.cs (open attribution). Awaiting operator on-air bench.

#194 logged (come-back-to): CWX/console CW wrong above ~27-29 WPM. Not a

clamp — two compounding timing causes: cwDitMs integer truncation (1200/wpm) + the wire cwx bit only moving at the ~2.6 ms EP2-datagram cadence (inherent to host CWX; high-speed CW → paddle/keyer in the KEY jack). Easy fix = round the timing; wire floor is structural. See task #194.

### Release

Version 0.7.0→0.8.0 (CMakeLists + lyra.iss), README features, USER_GUIDE

(companion launch + CW external/serial keyer), docs/releases/v0.8.0.md.

Bundle also carries the earlier unreleased main commits: #188 tuner-delete

trashcan (103c5fe), docking-feel smoothing (f31375a), #191 CAT virtual-COM enumeration (732b753), #189 layout save/recall fidelity (11dbed8), #190 USB-BCD cable lifecycle (27b7287), #158 VAC latency (dc9c4c2), docs (e199993).

## 2026-06-29 (EOD) — Post-v0.7.0: docs + VAC latency posture (VarAC turnaround)

All on main, unreleased (after v0.7.0). A doc pass + one operator feature.

### Docs (`e199993`)

Recorded the v0.7.0 FM transmit refinements as a TX-relevant entry in

THETIS_DIRECT_PORT_PLAN.md's SHIPPED-WORK LOG (FM is a TX modulator concern): fix #2 AF brick-wall b72bc50, FM-aware bandwidth 4f32ca1, pre-emphasis Off/Comm a826c8c, mic-rack auto-bypass 12c8991, Tuning-panel pre-emph chip + Dev→RX-BW 67abf05. RESUME POINTER + trailer refreshed to v0.7.0 (PureSignal + RX2/Split = remaining major TX-port arcs). Regen plan PDF/DOCX.

New docs/REMAINING_WORK.md (+ PDF): standalone what's-left checklist — much

smaller now (two big arcs left, RX2/Split + PureSignal, plus TX/CW follow-ons). Cleaned two stale tasks (CW macros #176, Spots #182 — shipped).

### VAC latency posture — `dc9c4c2` (#158 follow-up, operator-bench-confirmed

### "works better for VarAC")

Operator wanted to squeeze VAC TX↔RX turnaround for VarAC/ARQ (runs 25 ms

rings in Thetis). The IVAC engine already ported the full latency surface (SetIVACInLatency/OutLatency/PAInLatency/PAOutLatency + vac_size, driven by hardcoded vac1LatencyMs_=120/vac1VacSize_=2048); this exposed it.

Settings → Audio → VAC1 gains: Buffer size dropdown (128..8192, each

ms-labelled at 48 kHz), Latency spinbox (5..500 ms — rmatchV ring depth; one knob feeds all four reference latency fields), + a live Monitor line (ring fill % + overflow/underflow per direction, from getIVACdiags on a 500 ms QTimer — the reference VAC1 Monitor). Drop latency until counters twitch, back off.

setVac1LatencyMs/setVac1VacSize persist (vac1/latencyMs, vac1/vacSize)

+ reopen VAC when live via the proven rebuildVac1 path; vac1Diags() = guarded diag read.

Carried per-profile (Profile schema v4→v5, tolerant-load): a tight VarAC

profile keeps low latency while SSB stays at 2048/120 — flip profiles, flip the whole VAC posture. USER_GUIDE "Latency (for fast ARQ modes like VarAC)" note added.

GOTCHA: reference ivac.c:795 type comment is REVERSED vs its code; the

TO/FROM VAC diag labels follow the code (rmatchOUT=TO type0 / rmatchIN=FROM type1, Ivac.cpp:797). Flag if they read backwards at bench.

Operator decision: ACCUMULATE — stays on main unreleased; testers stay

on v0.7.0 until the next release bundles it.

### NEXT SESSION — operator-flagged (2026-06-29 EOD)

Tuner panel delete → trashcan icon. Cosmetic: the per-row delete control

on the Tuner panel (currently the − button, with the ✓/✗ confirm) should be a trashcan symbol. src/qml/TunerPanel.qml.

BUG — layout save/recall re-adds the Solar panel. Saving a named panel

layout WITHOUT the Solar (HamQSL) panel open, then recalling it, ADDS the Solar panel even though it wasn't present at save time. Only the Solar panel shows this so far — other panels round-trip their hidden state fine. Smells like the Solar panel has a default-visible/auto-show that the layout-restore path doesn't suppress (cf. the Tuner base-dock "shows on summon" pattern from this session — a dock that buildDocks creates visible/auto and the saved hidden state doesn't override on restore). Investigate the named-layout save/restore (#184 / v0.4.8) dock-visibility serialization for the Solar/HamQSL panel specifically. See [[reference-hamqsl-solar]].

**⚠ SAFETY BUG (task #190) — USB-BCD adapter intermittently undetected +

amp changed bands during RX.** FTDI C232HM (FT232H, COM1 in Windows) often not seen by Lyra's BCD dropdown; amp "beeped + changed bands in RX" (NOT RFI — heavily choked cable). Code-read root causes in src/usb_bcd.cpp: (a) enumeration is D2XX-only and the adapter is in VCP mode (owns the interface) → intermittent D2XX enumerate (the unplug/replug symptom); (b) devices() (~103-114) filters empty serials, but FTDI returns an empty serial for any OPEN device and Lyra's ctor opens+holds the device when BCD is enabled → its own dropdown shows "(none)" (matches the screenshot). Fix on the operator's bench (verify-don't-guess; spurious-BCD glitch is a separate lower-confidence item — get a repro + logging first). Interim safety: untick "Enable USB-BCD amp band output". Full detail in the memory note + task #190.

## 2026-06-29 — RELEASED v0.7.0 "Tuner memory" (Tuner panel + FM polish + tooltips toggle + header tidy)

Cut v0.7.0 off main (bump CMakeLists.txt + installer/lyra.iss 0.6.0 → 0.7.0; README Features → v0.7.0 + Tuner bullet; release note docs/releases/v0.7.0.md; USER_GUIDE "Tuner (manual ATU memory)" section + header-chip + TOC; regen PDF/DOCX). Installer dist/Lyra-Setup-0.7.0.exe.

Headline: the new Tuner manual-ATU memory subsystem (operator-requested 2026-06-29, benched this session).

src/tunermemory.{h,cpp} + src/qml/TunerPanel.qml + Settings → Tuner tab.

3 antennas × freq-keyed Input/Output/Inductor, live colour-coded SWR, nearest-point bracket view (1 each side; exact + 1 each side when on a point — windowEach=1), Store→Save, single-row inline edit, ✓/✗ delete confirm. Match by carrier freq (USB/LSB-agnostic). Pure UI + QSettings, RF-safe.

Tuner polish this session: SWR pill enlarged; flat (background-Item{})

header buttons; matchDeltaHz sign flipped to point − dial (so + = saved point above the dial — read the natural way).

Header chip strip regrouped: TX DSP · RX DSP · Options

(CTUN · Tuner · CW · CW Dec · WF-ID). CW/CW Dec/Tuner moved into Options; RX DSP slid left. mainwindow.cpp buildToolbar.

Also folded into v0.7.0 (committed since v0.6.0): FM pre-emphasis Off/Comm

selector + FM-aware bandwidth + Deviation→RX-BW auto-size + native mic rack auto-bypass on FM; global Show tooltips toggle (Settings → Visuals) + Profile-picker polish.

The earlier "float like TX DSP" experiment for the Tuner dock was reverted

(11dd6d2) after it caused intermittent launches; re-approached as just the isChipSummonedPanel predicate entry (297a753) which fixed the cyan dock-drop targets without the launch issue.

## 2026-06-29 — FM TX "beat Thetis" arc (pre-emphasis + clean chain + Tuning-panel UX)

All on main, unreleased (on top of v0.6.0). The FM transmit arc, brought to a complete stopping point. WDSP study-first: read fmmod.c + emph.c in full before touching shipped behaviour.

### FM Fix #2 — clean audio band-limit (`b72bc50`, earlier; logged for completeness)

The FM primary bandpass bp0 runs on the AUDIO before xfmmod, so it's

now a symmetric ±3 kHz brick-wall AF low-pass (was inheriting ±(dev+3k) ≈ ±8 k, over-feeding the modulator). The occupied-RF (Carson) clamp stays owned by the modulator's own internal bandpass, pinned via new SetTXAFMAFFreqs(txch, 300, 3000) on FM entry. Supersedes Fix #1's bp0 value (v0.6.0).

### Mode-aware FM bandwidth (`4f32ca1`, earlier) — `ModeFilterPanel.qml`

FM TX-BW selector + RX↔TX 🔗 lock disabled in FM (FM width is derived from

deviation). TX-BW box is a read-only "<N> k (auto)" Carson readout 2·(dev+3k), live-tracking Dev. FM RX presets = FM channel widths

8/10/12/16 k (was the SSB list).

### #3a — pre-emphasis selector Off / Comm (`a826c8c`)

WDSP-native via SetTXAFMEmphPosition (bound in wdspcalls X-macro):

Comm = position 1 = the native 6 dB/oct (300–3000) comms curve runs;

Off = position 2 = both call sites pass through = a TRUE bypass (FM force-runs the emph block, so the chain position is the on/off — a real Off, which rigs/Thetis don't give you).

HL2Stream fmEmphasisMode Q_PROPERTY (0=Off, 1=Comm, default Comm) +

QSettings tx/fmEmphasisMode + TxControl::setFmEmphasis callback; applied on FM entry in pushTxFilter + live on operator change.

UI: Settings → TX → FM combo (beside Deviation / CTCSS).

### #3b-3 — FM auto-bypasses the native mic rack (`12c8991`)

In FM the whole TX rack (EQ + Combinator + Plate + speech enhancements)

auto-bypasses, same forced-auto as the digital modes — a multiband compressor/reverb into FM's pre-emphasis = mush. One-line predicate change in main.cpp (txModeBypassesRack now fires for "FM" alongside DIGU/DIGL → SetTxRackBypass).

### Tuning-panel UX (`67abf05`) — `TuningPanel.qml`

Emph quick chip (Comm / Off) added to the FM front row beside Dev/RPT,

two-way-synced with Settings via the shared Stream.fmEmphasisMode prop.

Deviation now auto-sizes RX bandwidth to the smallest FM channel

preset ≥ Carson 2·(dev+3k) — ±2.5 k → 12 k, ±5 k → 16 k. TX BW was already deviation-derived; RX now follows (Thetis-style). Still overridable in the Filters panel.

### #3b 50/75 µs + 300 Hz HPF — DROPPED / DEFERRED (operator call)

Read emph.c: WDSP FM pre-emph is a 6 dB/oct ramp between f_low/f_high.

50/75 µs are broadcast-FM constants (15 kHz audio, ±75 kHz dev), meaningless over our 3 kHz comms band (75 µs corner ≈2122 Hz = a weak top-end lift; 50 µs corner ≈3183 Hz is *above* the 3 kHz cutoff → no pre-emph at all). "Comm" already IS the correct amateur curve → 50/75 µs

dropped (dead UI options). 300 Hz low-cut deferred — needs a new native HPF stage (can't ride bp0 = Hilbert trap; rack is bypassed in FM), mostly redundant with Comm's bass rolloff; build only if FM-data-flat bass-cut is later wanted.

### Docs

USER_GUIDE: FM section rewritten (deviation / pre-emphasis / CTCSS +

clean-chain note + Tuning-panel mirror + Dev→RX-BW); TX-BW + Lock bullets note the FM auto-width behaviour; Tuning-panel FM front-row bullets updated.

### UI cosmetics — global tooltips toggle + Profile picker (`ff0cdf9`)

Global "Show tooltips" toggle (Settings → Visuals, default on,

persisted ui/tooltips_enabled). New Prefs.tooltipsEnabled; all 69 QML ToolTip.visible bindings across 9 panels AND it in (swept), and an app-wide QEvent::ToolTip event filter (TooltipGate in mainwindow.cpp) swallows QWidget (Settings dialog / menu) tooltips too → one genuinely global switch. USER_GUIDE Visuals → Tooltips section added.

Profile picker (ProfilePanel): the recall tooltip no longer pops over

the open dropdown (gated !popup.visible); combo width 300 → 170.

## 2026-06-28 — v0.6.0 RELEASED: per-band TX power model + auto-calibrated watts cap + FM fix #1

Released v0.6.0 (5b41fc3; installer dist/Lyra-Setup-0.6.0.exe, 68.2 MB, GitHub release published; main FF'd). Bundled the whole TX power-model arc + the first FM fix that had been pending on main.

### Per-band TX power model (Settings → **PA Gain** tab)

ChannelMaster txgain.c ported → wire/TxGain.*; operator drive% drives a

digital fixed gain (Thetis RadioVolume) on top of the AD9866 byte, so 1 % drive is a true sub-watt (was pinned at the ~0.78 W nibble-0 dead zone).

Per-band PA Gain table (RadioVolume = min(drive·gbb[band]/100/93.75,1))

so the dial can be tuned to read true watts per band.

Watts Max cap (amp protection) + auto-calibrate: key TUN per band

into a dummy and Lyra walks power up to the cap and locks that band exactly (✓), SS-amp-safe (approach-from-below). Coloured Cap-tuned marks (green ✓ tuned / red — not tuned). The old "Max TX drive %" (#170) control retired.

### FM fix #1 (`ae6eb19`) — FM Carson bandpass clamp (value later superseded by Fix #2)

FM output bandpass sized from the FM signal's own occupied channel instead

of inheriting the wide SSB/ESSB TX width → no FM splatter at wide TX BW.

### Docs

USER_GUIDE "Settings → PA Gain" section; docs/releases/v0.6.0.md; README

bumped to v0.6.0.

## 2026-06-27 — v0.5.0 RELEASED: DX-cluster spots subsystem (#182) + CW RX-decoder overhaul (#187) + #159 DSP filter type

Released v0.5.0 (63b0799; CW 7ebc599) — two commits on main, installer dist/Lyra-Setup-0.5.0.exe (68.2 MB) published as a GitHub release. (v0.4.11 — the Kenwood CAT-over-COM/TCP + serial-PTT arc — shipped between v0.4.10 and this; it has its own release notes.)

### DX-cluster spots subsystem #182 — the headline (`spotstore.*`, `spothole_feeder.*`, `dxcluster_feeder.*`, `dxcc.*`, `PanadapterPanel.qml`, `settingsdialog.*`)

Source-agnostic spot bus (SpotStore): call-keyed dedup, age-out,

own-call highlight + toast, source/continent/firstSeen fields.

3 feeders, each its own on/off (Settings → Network → DX-cluster spots):

TCI inbound (logger / SDRLogger+) — addSpot(...,source="tci").

SpotHole REST (SpotHoleFeeder) — spothole.app/api/v1/spots,

standalone (internet only), default-OFF poll. Parse is snake_case (dx_call / freq in Hz / de_call / dx_continent / skip qrt) — first cut used hamlog's processed field names and got zero spots; fixed by probing the raw API.

DX-cluster telnet (DxClusterFeeder) — host/port/login-call

(defaults MYCALL, overridable) + login-commands, auto-login + auto-reconnect 20 s. NOTE: VE7CC-type nodes default to RBN CW skimmer only; set/ft8;set/ft4 login cmds open up FT8/FT4 (confirmed by reading the node banner — not a bug).

Filters (display-only; the bus keeps everything): modeShown

(family-aware CW/SSB/DIGI/AM/FM), regionShown + DxccLookup::continentOf ISO-2→continent table (continent codes NA/EU/AS/SA/AF/OC always mean continents, else ISO-2 country; "US,EU" = US + all Europe).

Clutter caps: per-freq bucket cap (freqHz/bucketHz bins, keep N

newest, "+K more" badge — tames FT8 piles) + global display cap.

Colour modes: source / single / by-mode / by-region / by-country +

optional legend (legendEntries() Q_INVOKABLE). New-spot colour: a fresh spot wears a solid colour for N s (driven by firstSeen, not added, so a re-poll doesn't reset it) then settles. Two bugs fixed here: spots flipping green↔gold every poll (separated firstSeen from added), and the original blink flashing EVERYTHING on startup (reverted blink → solid new-spot colour; renamed all "Flash" UI + docs → "new spot colour" so folks don't expect a blink).

Console import: click a spot → tune to it + CwMacros.hisCall grabs

the call into the CW-console contact row (#176/#181).

Band display filter DEFERRED (redundant on the panadapter; lands with a

future spot-LIST view).

### CW RX-decoder overhaul #187 (`CwDecoderPanel.qml`)

Decode-window scroll fix: ScrollView + `onContentHeightChanged →

Qt.callLater(scrollToBottom)` so rows stop clipping at the 3rd line before a resize. (Took two passes — the first "fixed" it only until the text reflowed.)

Narrow detect-filter BW chips tuned to [80,100,120,150]; Auto level

with live readout; Advanced (Seek / DSP filter / AFC range). Operator copy-confirmed on real Field Day CW (S4–S9, 10–25 wpm).

### Also shipped

CW console scroll (CwConsolePanel.qml): wrapped body in a

ScrollView so the macro tags + bottom option rows stay reachable when the console is sized small.

#159 DSP filter type (Linear Phase / Low Latency per mode family,

opt-in) — was the prior pending item; rode out with v0.5.0.

### Docs / release

USER_GUIDE: new "DX-cluster spots" section + reworked "Reading CW — the

RX decoder" section; #159 Linear Phase / Low Latency already present.

docs/releases/v0.5.0.md; version single-sourced via CMake

project(VERSION 0.5.0) → installer .iss + README.

Installer Excludes now drops test_*.exe (release hygiene).

## 2026-06-25 — v0.4.10 RELEASED: CTUN drag-the-marker (4th look, FIXED) + CW decoder A-series + CW-decoder docs

Working on main. Two releases since v0.4.8: v0.4.9 (fast patch — floating tool panels are float-only, Lock no longer disables the CW console/decoder; 162906f) and today's v0.4.10 (ebb3c67).

### CTUN #174 — 4th look, the pass that actually fixed it (`PanadapterPanel.qml`)

Operator (Thetis Help open as ground truth) said CTUN still wasn't Thetis

after the v0.4.4/v0.4.5 "parity" rebuilds: you should drag the FREQ MARKER onto signals with the display LOCKED; a click INSIDE the filter must NOT jump the 0-beat.

Read the real Thetis mouse handlers FIRST (console.cs 50230-50433 press +

51887-51960 drag). The DSP core (locked-LO + WDSP NCO shift) was already right; the whole gap was the MOUSE + MARKER layer.

ROOT CAUSE the 3 prior passes all missed: Lyra's carrier marker is

hard-pinned to the locked display centre (markerOffsetHz = CW pitch only), so under CTUN there was NO marker tracking the dial — nothing to drag. First bench report: "marker does not move."

Fix (CTUN-gated, non-CTUN byte-identical): new ctunDialOffHz slides the

carrier marker + filter passband to (rx1FreqHz − centre); body-drag = relative 1:1 marker-grab (setRx1FreqHz); top-15px strip = LO pan (setCtuneCenterHz); inside-passband click suppresses the retune; the edge-drag + inPassband math subtract the dial offset.

Operator-bench-CONFIRMED "Do as Thetis does state." #174 DONE.

### CW decoder — rest of the safe Tier-A A-series (A4/A6/A7/A8) (`CwDecoder.cpp/.h`)

A-slice A1/A2/A3/A5 was committed last session (eb4c89d). v0.4.10 adds:

A4 sub-bin AFC: 3-pt parabolic interp on the winning Goertzel bin

(refines bestHz only).

A6 hysteretic SNR gate: open at squelch×floor, hold to 0.8× that

(≈1.2× floor); inert on clean signal.

A7 unify the two word-gap emitters: guard the processEdge Word emit with

lastEmitted_ != ' ' so a word gap can't double-emit.

A8 add _ / $ punctuation; prosigns deliberately omitted (alias +/( ).

All additive — correct decodes unchanged, ~35 tuned constants untouched.

Unit test ALL PASS (VVV TEST CQ / CQ PARIS / CQ TEST; AFC lock 749).

### CW decoder docs (`USER_GUIDE.md`)

Operator couldn't recall what the decoder NB / DSP-filter did → realised

none of the decoder was documented (+ a stale "decoder coming in a later release" line). Added "Reading CW — the RX decoder" section + TOC entry: panel, confidence dimming, decoded-call/name grab → His Call / Name (feeds {CALL}/{NAME}), Match-TX-to-RX-WPM, and every detector knob. Console His-call cross-linked to the decoder grab.

### B1 anchor (NEXT CW work — NOT started)

Benched a live W1AW run; it announces its own speed ("END OF 30WPM

TEXT", "NOW 25 WPM") = labeled truth. Sent 30→25 wpm, decoder read 35 → over-reads ~15-20%, dit estimate runs short. Copy still readable; residual errors (WILL→WIRL, ONCE→JNCE) are valid-letter swaps A1 can't touch by design. W1AW = the labeled bench to tune B1 (independent dah-length + warm-up/Gaussian boundary fix).

### Release mechanics + standing

0.4.9 → 0.4.10 (CMakeLists / lyra.iss / README); notes

docs/releases/v0.4.10.md. Commit ebb3c67, tag v0.4.10, pushed main + tag, GitHub release + dist/Lyra-Setup-0.4.10.exe published.

Daily backup task ("Lyra-cpp Daily Backup", 22:30 → _backups/ + mirror

Z:\LyraBU\) confirmed ACTIVE + healthy (last run 6/24 OK, 0x0).

Open: CW B1 (with the W1AW bench). CTUN done. Bigger arcs still open —

RX2 #96-101, CW macro chips #176, grab-spot-as-call #182, PureSignal.

## 2026-06-24 — v0.4.8 RELEASED: panel docking overhaul + CW transmit metering (+ wiki, CW-decoder review)

Working on main (last tag v0.4.7 = ff6c9c9). Tester feedback (Randy W7CPA / Jim / a 3rd) said the main panels "all float, nothing docks, drop-against-an-edge does nothing," and resizing was a fight. Big UI arc + a CW metering arc, shipped as v0.4.8.

### Panel docking overhaul (#184) — `dockdragcontroller.{h,cpp}` (NEW) + `mainwindow.{h,cpp}`

Root cause: the custom dock title bar (makeDockTitleBar) replaces

QDockWidget's built-in drag, killing Qt's drag->drop-indicator->snap-to-edge.

New DockDragController (public QMainWindow API only): press->threshold->

drag->drop FSM on each title bar, translucent cyan drop-zone overlay — snap-to-edge (addDockWidget), neighbor split L/R/T/B thirds (splitDockWidget, "before"=double-split), center tabify (tabifyDockWidget), off-region float (setFloating). Save-on-commit.

Resize: dock separators widened 6px + cyan-on-hover; tab bar styled.

Compact custom move cursor (arrow while locked). One-time status-bar hint.

Lock = move + resize: disables move/float/close via features AND blocks

the dock-separator resize press in event() (split-cursor detect). The earlier setFixedSize-on-lock froze panels and collapsed the layout on a locked restart — REMOVED; event-interception locks resize cleanly and always restores. (Two intermediate restart-collapse symptoms chased + fixed.)

View -> Layouts: 4 named slots + Lyra default (recall no longer

force-shows the chip-summoned TX/RX/CW rack panels).

### Factory default fixed — `default_layout.h` + `applyDefaultLayout`

Prior embedded default was base64 of the raw registry REG_BINARY -> carried

the QSettings @ByteArray(...) UTF-16 wrapper -> restoreState() rejected it (the "screwed up default"). Re-embedded from operator's N8SDR slot, de-wrapped to the real saveState bytes, + defaultPanadapterSplit().

### CW transmit metering (#105) — `hl2_stream.{h,cpp}`, `metermodel.{h,cpp}`, `main.cpp`, QML

Bug: QSK CW keys the PA at the gateware and deliberately leaves wire MOX

low, so the meter (gated on moxActive) never flipped to TX during CW (external watt-meter showed power). Other modes flip because they assert MOX.

cwKeyingActive (message-level, from the CwKeyer onState hook, marshalled

to the stream thread) drives the meter RX<->TX swap + the Ladder source set. Reads fwdPowerW() (already valid during gateware CW). Does NOT assert wire MOX — QSK keying unchanged.

txDisplayActive (= moxActive || cwKeyingActive) drives the red-on-air

QML (VFO border + RX/TX tag in TuningPanel, split/XIT TX markers in PanadapterPanel).

Paddle/key detection: onHwPttPoll (~50 ms) latches cwKeyingActive

from forward power (>=0.5 W, ~500 ms hang), gated to CW mode, skipped while CWX drives it — this HL2+ reads non-zero ptt_in at rest (§10 Q#1), so forward power is the clean signal.

The panadapter spectrum stays RX during CW (gateware CW never feeds the

WDSP TX analyzer -> swapping shows an empty pan; RX shows the keyed carrier). Operator-confirmed regression + fix.

### Release / docs / wiki

Version 0.4.7->0.4.8 (CMake project(VERSION) + installer/lyra.iss).

docs/releases/v0.4.8.md; USER_GUIDE "Getting around the window" rewritten + Meter CW note; README Features->v0.4.8. ISCC -> dist/Lyra-Setup-0.4.8.exe (69 MB). Tag v0.4.8, pushed main+tag, GitHub Release published w/ installer.

GitHub Wiki initialized + built: Home, User Guide (mirror of

USER_GUIDE.md), Installing-and-first-connection, FAQ-and-Troubleshooting, _Sidebar. Sources version-controlled in docs/wiki/ (User Guide generated from USER_GUIDE.md at publish). NB: the first wiki page must be created once in the web UI (no API, no push-init) — operator did it; rest pushed via …wiki.git.

### CW decoder accuracy review (analysis only — NOTHING changed)

Recovered the 4-reviewer + code-cross-checked review (was transcript-only)

and committed it: `docs/CW_DECODER_REVIEW.md` (Tiers A–D). Target = src/dsp/CwDecoder.cpp/.h (field-tuned constants).

Operator chose the safe A-slice = A1+A2+A3+A5 to do next (nearest-code

? rescue / AFC center-bias / per-char confidence / crash-proof peak-snap — none touch the tuned constants; rule: never corrupt a correct decode). Then B1 (independent dah-length + the √3-vs-1.565 boundary discontinuity). NOT STARTED. See [[project-lyra-cpp-cw-decoder-accuracy]].

## 2026-06-23 — v0.4.6 RELEASED: connectivity + multi-radio + front-panel polish

Working directly on main (last release v0.4.5 4a106d5). Connectivity/ polish release — no wire/TX/audio-path changes; all UI / discovery / installer.

### Installer (lyra.iss)

Auto Windows Firewall inbound rules for lyra.exe: UDP (HL2 discovery

reply + EP6 RX stream) + TCP (TCI server, future networked clients). Named "Lyra SDR", delete-then-add (idempotent), removed on uninstall. Fixes the "needs admin to connect" report — the unsolicited inbound discovery reply was being dropped for a non-elevated launch. Matches Thetis's WiX FirewallException approach.

Opt-in `NetworkThrottlingIndex=0xffffffff` task (default UNticked) — the

classic multimedia network-throttle "gaming" tweak; [Registry], left in place on uninstall. Mirrors Thetis NetworkThrottle.cs.

### Discovery (hl2_discovery.{h,cpp})

Dual broadcast: sendBroadcastFromAllSockets now sends to both

255.255.255.255 AND each NIC's subnet-directed broadcast (entry.broadcast()).

`probe(ip)` — directed unicast discovery for Add-by-IP (own socket +

1 s deadline, reuses parseReply→radioFound). Independent of the sweep.

### Settings → Hardware radio panel (settingsdialog.cpp)

Add by IP fires probe() → the placeholder upgrades in place to the real

board/gw/rx (addRadio changed skip-on-dup → update-in-place).

Double-click a row → Open; connected radio shown green + bold

(item roles, survives re-render); one-line helper.

### Front panel (mainwindow.{h,cpp})

Start green / Stop red (backing QToolButton), **Connected green /

Disconnected red / Connecting-Scanning amber** (connStatus_).

Header chip strip: new Options: separator+label so CTUN / WF-ID

no longer read as RX-DSP functions.

### Release

Version bump CMakeLists 0.4.5→0.4.6 + lyra.iss AppVersion. Release note

docs/releases/v0.4.6.md; USER_GUIDE header + Radio sections; README Features (v0.4.6). Tag v0.4.6, installer rebuilt, pushed to origin/main + GitHub release.

## 2026-06-22 — v0.4.5 RELEASED: CTUNE rebuild + Waterfall ID + CW macros

Branch feature/ctune-rebuild-wfid; main was at v0.4.4. This is the v0.4.5 release.

### #174 CTUNE rebuilt (operator: v0.4.4 "built wrong")

4-agent Thetis-vs-Lyra audit. Centre/shift/edge decision relocated from QML

into the C++ RX-NCO chokepoint (HL2Stream::pushEffectiveRxFreq) so EVERY tune source behaves (dial / click / keypad / band+memory / TCI spot): smooth- scroll at the 5% display margin, far-jump re-centre, zoom force-centre, IQ- Nyquist clamp. RXANBPSetShiftFrequency added so notches/NB track the shift.

Commit 29acc6d (design docs 0e0f2f2).

### #175 Waterfall callsign ID

TX courtesy ID: the call rendered as a readable image in the SSB passband.

Header arm chip (USB/LSB only), Settings → TX config (level ≤0.065 / repeat / over-drive warning), crash-safe transient flat-TX orchestrator (TCI source + whole-rack bypass + DIGU/DIGL, restore on un-key).

LSB mirror bug fixed this session: DIGL flips audio↔RF, so the call came

out mirrored in LSB (USB was fine). Renderer now pre-mirrors the frequency layout for LSB → reads upright on both sidebands. Commit 79d8d43.

### #176 CW macros — named click / F-key memories

Redesigned floating CW console into a contest-grade macro bank: named click +

F-key (F1–F12, global) memories, a "My macros" group the operator builds out, live "sending" state, Repeat, a contact row (His call / Name / RST / #).

Tokens: contact {CALL}/{NAME}/{RST}/{#} + {MYCALL}, plus operator personal

tokens ({ME}/{PWR}/{QTH}/{RIG}/...), with a click-to-insert palette.

New CwMacroModel persists macros / tokens / serial to QSettings. F-keys via

MainWindow::keyPressEvent (QML Shortcuts don't fire in the QQuickWidget docks). Commit 9f0d054.

### Settings dialog

Per-tab scroll-area + larger default size (long tabs were clipping).

### Release

Version bumped (CMakeLists.txt + installer/lyra.iss → 0.4.5), release note

docs/releases/v0.4.5.md, User Guide CW section updated, annotated tag v0.4.5, installer dist/Lyra-Setup-0.4.5.exe, pushed to GitHub; main fast-forwarded to the release.

## 2026-06-21 — v0.4.3 RELEASED: 60 m band plan + tune-drive modes + BCD polish

Operator quality-of-life batch + release. All on main (project commits releases directly to main, tags + GitHub release + installer). Released mid-afternoon, then held for the day.

### 60 m band plan (`bandplan.{h,cpp}`, `PanadapterPanel.qml`)

Clickable 60 m channels: US CH1–CH5 now render as always-visible

labelled bars (a touch larger), each a MouseArea that QSYs (Prefs.mode= "USB" + Stream.setRx1FreqHz(qsy)) to the channel's USB dial — same one-click idiom as the FT8/FT4/PSK watering-hole markers. segments() flags channel/qsy per sub-band; delegate gained chan prop + conditional label size/visibility + tooltip.

Region/country-correct 60 m: precise US channel dial freqs

(5330.5/5346.5/5357.0/5371.5/5403.5 kHz); WRC-15 contiguous band (5351.5–5366.5) added to r1Bands() (was missing). New Country override layer: regionBands(region) + countryBands(country) + bandsFor(region, country) merge (replaces same-named band in place). BandPlan::country() + bandPlanCountryChanged→regionChanged; all 3 bandsFor call sites updated. UK = its 11 RSGB/Ofcom segments; Canada = WRC-15. Prefs.bandPlanCountry (key band_plan/country, default "AUTO", validated vs {AUTO,UK,CA}). Settings → Hardware Country combo added.

Verify-don't-guess applied: UK 11-segment list WebFetch-confirmed

(rsgb.org); AskUserQuestion before writing international freqs.

### Tune-drive selector — #95 (`prefs`, `main.cpp`, `Profile`, settings, TxPanel)

Replaced bool useTuneDrive with enum tuneDriveMode

{Slider=0/Tune=1/Fixed=2} + fixedTuneDrivePct (default 25), one-time QSettings migration (legacy bool → 0/1). main.cpp tune orchestrator rewritten to a tunePctFor() lambda + mode-aware arm/release (fixes a latent mode-switch-mid-tune restore edge). Profile capture/apply + TxPanel visible: Prefs.tuneDriveMode === 1 updated.

Max-drive cap is honored: wire chokepoint setTxDriveLevel

(std::min(maxDriveRaw(), …)) already covers TUN; added cosmetic defense-in-depth — Tune/Fixed spinners setRange(0, maxDrivePct()) + live follow on maxDrivePctChanged. So TUN can't exceed the operator's Max-TX-drive ceiling.

### 11 m → 10 m BCD option (`usb_bcd.{h,cpp}`, settingsdialog)

New elevenAsTen (key hw/bcd11as10, default ON), mirroring the

existing sixtyAsForty. Substitution lives in reapply(): an 11 m/CB freq (cbBandIndexForFreq>=0, amateur table returns -1) remaps to the 10 m amateur index → emits BCD 9. Settings → Hardware checkbox.

Plus an amber regulatory advisory under the BCD options: "Operate only

within the maximum power and band limits permitted by your country / region's regulations." (Operator-requested — states it plainly at the point of use re: 11 m abuse.) USER_GUIDE BCD section mirrors it.

### Docs + release

USER_GUIDE: Band-plan (Region+Country+60 m channels), Settings → TX

(Tune-drive subsection + Max-drive cap note), USB-BCD (11 m option + advisory). New docs/releases/v0.4.3.md (= GitHub release body).

Version 0.4.2 → 0.4.3 (CMakeLists.txt + installer/lyra.iss); full

rebuild (LYRA_VERSION embedded, USER_GUIDE resource recompiled) + ISCC → dist/Lyra-Setup-0.4.3.exe (68 MB).

06921f2 Release v0.4.3 (16 files, +598/−158) + the already-on-main

help-doc commit 4e62bb9 (log-capture section) rode out together. Tag v0.4.3, pushed main (ddd6677..06921f2) + tag, GitHub release published with installer attached + verified.

### Standing rules in effect

NO PATCHING (tune-mode done as a proper enum + migration, not a shim).

Commit/push only when asked — operator explicitly requested this release.

Verify-don't-guess for regulatory data (60 m freqs, UK segments).

## 2026-06-19 — v0.4.1 RELEASED: TX protection (SWR cut + drive cap) + PHROT

Short session: shipped the TX-protection arc, a PHROT parity feature with a digital auto-gate, doc updates, then cut + pushed v0.4.1 (main FF'd, tag + GitHub release + installer). All on main.

### TX protection — #169 SWR + #170 drive cap (SHIPPED + operator-bench-confirmed)

#169 SWR Cut (6c3e8e7): a 50 ms QTimer (swrEvalTimer_) armed on the

MOX edge in HL2Stream decodes prn->tx[0].fwd/rev_power → ρ=√(rev/fwd), SWR=(1+ρ)/(1−ρ) (calibration-free). Trips through the single requestMox(false) unkey funnel above a user threshold (default 5:1). Four false-trigger guards: blank window, fwd/rev power floors (NaN-safe), dwell. Latch + reason + PROT lamp on TxPanel (gray/green/red, elided so it can't overflow the ATT lamp). Settings → TX group (enable + limit + Cut/Fold + fold-floor + during-tune).

#169 Phase 1b Fold: optional action — steps drive ×0.5 via

applyDriveLevelNoPersist (no QSettings write, fold restored next keydown) and escalates to a hard trip at the floor.

#170a drive cap: maxDrivePct (1..100%) clamps every drive write at the

one chokepoint setTxDriveLevel (std::min(maxDriveRaw(), …)) — covers the operator slider, TUN, BandMemory restore, TCI DRIVE alike, + re-clamps live on lowering. Operator bench-confirmed ("ran slider max down, couldn't pull more"). #170b over-power trip DEFERRED by operator ("fine as it is").

Design doc docs/architecture/tx_protection_design.md stamped COMPLETE.

### PHROT — #109 phase rotator toggle + digital auto-off (SHIPPED)

WDSP TXA SetTXAPHROTRun exposed via the wire/wdspcalls X-macro seam +

a TxControl.setPhrotRun callback (d99794d). phrotEnabled Q_PROPERTY (Settings → TX checkbox, default ON = WDSP/reference posture). Parity with the reference Setup PHROT control — it is fully exposed there, not a hidden knob (operator asked; verified).

Auto-off in digital (165e176): one chokepoint applyPhrotRun()

computes run = phrotEnabled_ && !(txMode==DIGU|DIGL) (WDSP mode 7/9) and pushes via the callback — called from the operator setter, every `setTxMode` edge, and registerTxControl (channel open). The checkbox is now the operator's *voice-mode* intent; DIGU/DIGL switch it off on the wire and back on in voice modes (mirrors the native-rack SetTxRackBypass gate and the RX EQ #59 mode gate).

USER_GUIDE: new Phase Rotator section incl. a **"PHROT and wide / ESSB

audio (4–10 kHz)"** note (helps punch on comms-grade wide SSB; all-pass group delay smears hi-fi ESSB → many ops leave it off; A/B on own voice).

### Release

v0.4.0 already shipped, so this is v0.4.1. Bumped CMakeLists +

installer.iss, rebuilt clean, ISCC → dist/Lyra-Setup-0.4.1.exe (68.1 MB).

Commits 6c3e8e7 (SWR/drive-cap/#172/docs) → d99794d (PHROT toggle) →

165e176 (PHROT digital auto-off) → eff9a5d (version bump). Pushed main (1c6de15..eff9a5d), tag v0.4.1, GitHub release w/ installer.

One snag: LNK1104 (operator had Lyra running) — asked operator to close,

relinked clean (the standing don't-force-kill-operator-Lyra rule).

## 2026-06-18 — CW TX ON AIR: paddle + keyboard console + CW-over-TCI (#105)

CW transmit went from "design mapped" to fully on-air this session.

### CW-2 paddle (gateware iambic) — SHIPPED + operator-confirmed

applyCwKeyerEnable arms prn->cw.cw_enable in CW mode (CWL/CWU) — the

firmware-keyer + sidetone master; paddle in the HL2 KEY jack keys the carrier autonomously (gateware CWTX), host stays out of timing.

Carrier-on-marker fix: keyed carrier rides the marker in both sidebands

(single shared cwPitchHz); the off-marker was a DISPLAY bug (host MOX off the gateware key line switched to the TX analyzer) — fixed by NOT forwarding host MOX in firmware-keyer CW (QSK). Break-in selector QSK(default)/Semi/Manual; CW MON sidetone slider on the Audio panel.

Manual break-in foot-switch confirmed ("tone footswitch and paddle"):

foot switch = PTT hold (gateware PTTTX), paddle keys within (→CWTX).

Vestigial cwSidetoneFreqHz path removed (14be1ed) — CW pitch is the

single freq source.

### CW-3a/3b host software keyer (CWX) + CW console — SHIPPED `8904749`

Gateware-verified (radio.v / dsopenhpsdr1.v): host CWX keys the carrier

via the SAME CWTX path as the paddle — host drives tx[0].cwx (per element) + cwx_ptt (held per message); cw_enable (CW-mode armed) enables BOTH the host EP2 overlay AND the gateware CWX decode (cmd_data[24] = the 0x0f C1 bit). NO WDSP / no host carrier gen. The earlier "dot/dash bit" guess was WRONG — tracing corrected it.

tx/CwMorse (table + PARIS timing + operator weight, pure/testable),

tx/CwKeyer (dedicated-thread element pump, monotonic absolute-deadline scheduling, interruptible abort), HL2Stream sendCw/abortCw + setCwxKey/setCwxPtt + lazy keyer + abort-on-close.

qml/CwConsolePanel — chip→floating console (audio-rack idiom): WPM,

type+Enter sends / Esc aborts, Send+Stop, reserved CW-5 decoder pane. "CW" launcher chip after the TX DSP strip.

Design doc docs/architecture/cw3_software_keyer_design.md.

### CW-4 CW-over-TCI — SHIPPED `fcfcb6b`, EESDR-spec-verified

tci_server dispatch routes the EESDR TCI CW commands into the CW-3

keyer: cw_macros (plain send), cw_msg (contest prefix/call/suffix + $N repeat), cw_macros_speed/cw_keyer_speed (WPM, bidirectional), cw_macros_stop. Reserved-char un-escape ^~*→:,;. sendCw/abortCw marshalled to the stream thread (QueuedConnection) — no race vs the console. CW-mode-only.

Verified vs the EESDR TCI protocol manual (operator added

docs/TCI Protocol.pdf §3.2) + Thetis TCIServer.cs + SDRLogger+. The EESDR cross-check caught 3 drafting errors: cw_macros_empty is a server→client terminal signal (not a stop), reserved-char un-escape was missing, and cw_msg (the canonical contest cmd) was missing.

### Docs

docs/help/USER_GUIDE.md (2a09824): new "CW operating" section

(paddle/console/TCI + break-in + CW MON) + "CW keying over TCI" + TOC.

### Standing directive (recorded)

TCI work verifies against BOTH Thetis TCIServer.cs AND the EESDR TCI

manual (docs/TCI Protocol.pdf) — the authoritative spec.

### Next

CW-5 (#173) — RX CW decoder (faithful port of SDRLogger+'s

Bayesian/AFC/Farnsworth decoder, hamlog/cw.html) into the console's reserved pane + a macros field. Deep-read the decoder first.

CW follow-ons: Semi/Manual host-MOX for CWX, F-key memories, CWFWKeyer

toggle; TCI cw_terminal/cw_macros_delay/|..|+<> macro syntax.

On-air TCI CW bench via SDRLogger+ still TODO (spec-verified + builds).

### Commit summary (today)

2a09824 docs(cw): USER_GUIDE — CW operating + CW-over-TCI
fcfcb6b feat(cw): CW-4 — CW keying over TCI, verified vs EESDR TCI spec
8904749 feat(cw): CW-3a/3b — host software keyer (CWX) + CW console
14be1ed refactor(cw): remove the vestigial cwSidetoneFreqHz path

## 2026-06-17 EOD — v0.4.0 RELEASED + main consolidated + CW TX mapped

### UI readability batch (shipped `8fe2100`)

TX DSP launcher chips (Speech/EQ/Combinator/Plating) → boxed buttons that

fill with the accent when their panel is open (were flat text-only).

EQ graph: frequency-marker row across the top; x-range tracks the TX audio

passband (SSB/digital = high edge, AM/DSB/FM/SAM = half occupied) + amber dashed TX-edge marker; gain scale bigger (bold 11px) + brighter, labelled every 3 dB (added ±3/±9 rows), 0 dB bright blue.

Speech/Combinator/Plate parameter labels + readouts brighter + bold 13px;

Combinator per-band micro-labels 9/10→11px. Operator: "easier on the old eyes." .gitignore += _run_lyra_vac1.bat, Line.

### main consolidated (operator request)

origin/main fast-forwarded a3a517f..8fe2100 (clean, 32 commits, no

foreign divergence). Local main moved up + checked out — `main` is the working trunk again (TX largely done; stops the "forgot to FF main" risk). tx-rebuild kept as a legacy ref.

### v0.4.0 RELEASED

Version bump 0.3.1→0.4.0 (CMakeLists + installer/lyra.iss); release notes

docs/releases/v0.4.0.md; commit 88f17f2. Full rebuild (version baked), Inno Setup installer dist/Lyra-Setup-0.4.0.exe (~68 MB). Annotated tag v0.4.0, pushed main + tag, GitHub Release published with installer attached + verified. main==origin/main==v0.4.0==88f17f2.

### #156 (restart-after-hard-kill) — PARKED

Operator empirical: a real accidental hard kill of lyra.exe mid-VOICE-TX

(my pre-build force-kill fired while they were tuning) → restarted CLEAN. Only ever one observed hang (2026-06-13 mid-TUN). Intermittent → parked until it reproduces; prime suspect noted (infinite hReadThreadInitSem.acquire()), optional bounded-timeout hardening if revisited. PROCESS RULE logged: don't force-kill lyra for a build when the operator may be on air — ask first.

### CW TX (#105) — DESIGN MAPPED (rainy-night research, no code)

2-lane reference dive (Thetis host cwx.cs/console.cs/TXA.c + HL2 wire

networkproto1.c + operator's HL2+ gateware RTL). Wrote

`docs/architecture/cw_tx_design.md` — full port plan.

THE finding: HL2 CW is 100 % gateware-keyed (no host/WDSP carrier;

TXA.c SetTXAMode has no CWL/CWU case; radio.v:1145 bypasses EP2 TX-I/Q during CW, feeds the FPGA envelope ramp; FPGA also makes the hardware sidetone). Host just sends key-state. The verbatim EP2 CW packer is ALREADY in tree (NetworkProto1.cpp:99-108, dormant) + prn->cw seeded; EP6 dot/ptt decode already avoids the reference's left-shift bug. Missing: CW C&C composer cases (0x0f/0x10/0x0b) + the native CWX keyer engine + FSM CW PttSource + UX. Build order C-1..C-5 in the doc. ⚠ flagged: 0x0b register collision with the shipped ATT-on-TX step-att — resolve before composing.

## 2026-06-17 (cont. — AM/DSB/FM native TX modulation + AM carrier #106/#93)

### #165 follow-on → basic WDSP-native AM / DSB / FM transmit

Operator: keying AM showed only the upper sideband. RX was correct; the one-sided signal was the operator's own TX — the modulator was SSB-only.

`hl2_stream.cpp` `setTxMode`: widened the WDSP-mode clamp 0..1 →

0..13 so AM(6)/DSB(2)/FM(5)/SAM(10) reach SetTXAMode instead of being pinned to USB.

`main.cpp` `wdspTxModeFor`: full mode map (LSB0…SAM10, default USB),

replacing the AM/FM/DSB/SAM→USB collapse.

`main.cpp` `pushTxFilter`: per-mode sign-coded bandpass (TX mirror of

the RX §14.2 convention) — USB-side +low..+high, LSB-side -high..-low, double-sideband symmetric. NEVER calls SetTXABandpassRun (§15.23 trap); SetTXABandpassFreqs + SetTXAMode configure bp0. FM CTCSS silenced via SetTXACTCSSRun(ch,0) (TXA defaults it ON at 100 Hz).

TUN ∓cw_pitch offset generalized in txDdsHzForTune /

txAnalyzerOffsetHz so double-sideband modes stay centred.

New wire bindings: SetTXACTCSSRun, SetTXAAMCarrierLevel

(wdspcalls.{h,cpp} — pointer + X-table + extern).

### Doubled-bandwidth bug (operator bench) — FIXED

AM straddled centre correctly but occupied ~2× the set TX BW. Cause: the symmetric branch used ±high = 2×high total. txf->high is the set TX BW (Prefs.txBandwidth); a double-sideband signal occupies the full ±span, so the edge must be ±high/2 — mirroring WdspEngine::computePassband (half = bw/2, ±half) which also draws the filter markers. Now AM at 6k = ±3k = 6k total, inside the markers. Operator bench: correct.

### AM carrier level control (#93) — operator-facing, reference-faithful

HL2Stream amCarrierPct Q_PROPERTY (+ getter/setter/signal/member,

QSettings tx/amCarrierPct), TxControl.setAmCarrierLevel callback, main.cpp lambda → SetTXAAMCarrierLevel. Settings → TX "AM Carrier" spinbox (0–100 %).

Mapping matches the reference exactly (verified at

setup.cs:9846): UI is % of standard carrier POWER, coefficient = √(pct/100) × 0.5, default 100 = standard AM (= 25 % of PEP). So operator-stored values transfer 1:1. Helper amPctToCarrierLevel(), applied at both push sites. (Earlier draft passed pct→coeff straight, default 50 — corrected before bench.)

N8SDR's exported profiles confirm the intent: 8K-AM-N8SDR=40,

*-N8SDR-AM=84, AM 10k CFC=95 (reduced/controlled carrier).

### Profile import — decided NOT to pursue (operator call)

DSP chain is Lyra-native (EQ/Speech/Combinator/Plate, not WDSP/CFC), so importing reference profiles is pointless — the *sound* lives in the native rack, which has no reference equivalent. amCarrierPct NOT added to the Profile struct. Profile manager stays as-is.

### Status

All built clean + operator-bench-confirmed (AM/DSB/FM both sidebands inside the markers; AM carrier behaves). Committed this session; #106/#93 closed.

## 2026-06-17 (cont. — doc reconciliation + ATT-on-TX UI §15.31)

### Doc reconciliation (operator-delegated while out)

THETIS_DIRECT_PORT_PLAN.md reconciled (agent pass + my accuracy

corrections): obbuffs/P1 → [DONE]; compress/cfcomp/eqp → [N/A] (native-superseded); wcpagc leveler/ALC → [DONE] but softened — metering "shipped in code; #160 bench close-out + Brent/Timmy field-confirm still IN PROGRESS" (not over-claimed). PDF + DOCX regenerated via the generalized tools/sync_execution_plan.py (now --md/--docx/--pdf-path/--title/--no-docx). SESSION_LOG.pdf regenerated.

### §15.31 — ATT-on-TX operator surface + visible lamp (operator-flagged)

Operator: "ATT on TX doesn't appear to be live — I don't see the LNA drop to −31 on TX." TRACED the path (did NOT re-assert from the doc — the [DONE] I'd let stand was operator-disproven):

WIRE path IS live + correct: fsmAdvance raises

setTxStepAttnDb(31) on keydown (hl2_stream.cpp:1651) → tx_step_attn=0 → XmitBit-gated compose_case_11 C4 = 0x40 → ak4951v4 cmd_addr 0x0a rx_gain = 0 = min LNA during TX. The front end WAS being attenuated.

Root of the report = VISIBILITY gap (operator confirmed "1 and 3"):

the LNA readout shows the RX setpoint (unchanged on TX); no on-screen confirmation, where Thetis visibly jumps the ATT to 31. Also the src/tx/AttOnTxPolicy.{h,cpp} class is unused / wire-inert — the live mechanism is inline in hl2_stream.cpp; the doc citation was wrong.

FIX (operator-locked design confirmed before code; matches Thetis

Setup → General → Ant/Filters → "ATT on Tx" ✓ / "ATT: 31"):

HL2Stream: attOnTxEnabled_ (default ON) + attOnTxDb_ (default

31), Q_PROPERTY + setters + QSettings (tx/attOnTx*); fsmAdvance gates on the toggle + uses the value (replaces hardcoded kAttOnTxDb); mid-TX toggle re-applies live. Disabled → axis 0 (= reference SetTxAttenData(0)).

TxPanel.qml: "ATT" lamp in the gap between the Mic and Tune sliders,

AUTO/TUN/MOX lit idiom — gray ATT off / orange ATT 31 armed (RX) / solid-red + white-text ATT -31 engaged (keyed). Engaged colour fixed from a salmon-on-maroon blend that read PINK → MOX-style solid red; label shows the negative (applied attenuation) on TX per operator request.

settingsdialog.cpp: Settings → TX → "ATT on TX (RX-ADC protection)"

group (left/safety column) — Enable + ATT dB (0..31) spin, bidirectional with the lamp.

Builds clean; operator-approved the UI in review. THETIS row kept

[WIP] (flips [DONE] only on the on-air bench confirm). AttOnTxPolicy class still unused (flagged for later delete-or-wire-up). Task #114 (broader: panadapter offset + PA-enable safety) stays pending.

## 2026-06-17 (build day — #162/#163 ship, #90 TX monitor 3-route, #164 Out picker; all HL2-confirmed)

Branch tx-rebuild (NOT pushed; main/origin stays v0.3.1). Every commit below was operator-bench-confirmed on the HL2+ before it landed.

### #162 TX Speech profile NOT saving — FIXED (root cause CORRECTED vs the 06-16 theory)

The 06-16 "binding-break / UI-refresh" theory was WRONG. Operator pushback ("are we even capturing the Speech panel?") + a registry dump (HKCU\Software\N8SDR\Lyra-cpp\profiles\item\<name>) proved it: the speech blob saved all-default. Real root cause = QML signal shadowing — the Stage card's custom signal toggled(bool) was shadowed by AbstractButton.toggled, so the ON button's onClicked: toggled(checked) emitted the Button's signal, never the model write → Speech always saved defaults.

c17960c: qualify the emit stage.toggled(checked) → the model write

fires; registry-verified (deessOn:true … now persist).

46a428f: Binding{target:en; property:"checked"; value:on} re-asserts

the toggle lamp from the model on recall (a checkable Button flips its own checked on click, severing the inline binding).

Operator-confirmed: toggles save, recall re-lights lamps + restores

sliders, 2 fresh profiles round-tripped. Lesson: clean build + qmllint ≠ proof — verify by registry dump / launch.

### #163 Rack-lamp dim in digital/CW + EQ analyzer recolor (cosmetic, operator-requested)

9be1fc1: Speech/Combinator/Plate panels gray the ON lamp + dim controls

+ show an amber "bypassed (MODE)" header hint in DIGU/DIGL and CW (digital → SetTxRackBypass; CW → no mic). Purely visual off a WdspEngine.mode binding; model untouched, re-lights on USB/LSB.

558d5d9: EqPanel — same dim + the operator's analyzer recolor:

Accumulate peak-hold → RED, pre-EQ "Before" overlay → WHITE (were near-white/amber), live "After" stays cyan. Line + RTA. Doc 66adc5e.

### #90 TX audio monitor — ALL THREE reference-faithful routes SHIPPED (#90 closed)

ONE shared post-rack tap (xcmaster case 1, READ-ONLY on pcm->in, after the rack / before fexchange0) → lock-free SPSC MonitorRing (src/dsp/MonitorRing.h); drained ONCE at the top of dispatchAudioFrame into monScratch_, shared by all routes. WdspEngine monEnabled_/ monVolume_ (atomics, persisted); Audio-panel MON TX toggle + Monitor slider.

Route 1 (HL2 jack, 6e6a282): monitor mono on L/R in place of the

auto-muted RX when MOX up + MON on, via the existing OutBound(0) path.

Routes 2+3 (e1fd186): Route 2 = VAC stream-2 + SetIVACmon/

SetIVACmonVol (ivacGet-gated, re-applied in rebuildVac1); Route 3 = TCI RX-audio (tap moved into dispatchAudioFrame; MON→monitor / TX→mute / RX→live).

Reference-VERIFIED (operator asked "is this Thetis or have we drifted?"):

Thetis audio.cs:386-404 drives SetIVACmon(0,1)+SetIVACmonVol(0, monitor_volume) on the PRIMARY VAC (id 0 = our kVac1Id) and SetTCIRxAudioMox/Mon (368-404) — the single MON drives jack + VAC + TCI, exactly as built. NOT drift.

⚠ Faithful behavior change: TCI RX-audio is now MUTED on the air (was

live RX during TX). Operator-bench-confirmed harmless — MSHV FT8-over-TCI decoded + completed a QSO (N8SDR↔KB6DAD to 73) with MON TX on, TX 5.1 W, DIGU rack auto-bypassed. Docs 266d8cd.

### #164 "Out" output-device picker wired + MON→MON TX relabel (`740f769`, doc `dc45485`)

The disabled placeholder "Out" button is now a live one-click output picker: a styled top-level popup (popupType: Popup.Window so it ESCAPES the short Audio-panel QQuickWidget clip — a plain Item-type popup was getting chopped at the panel bottom with no scroll). Lists HL2 jack + PC devices, current ▶/cyan/bold, hover highlight, as-needed scrollbar. Reads the same audioOutputDevices()/setAudioOutputDevice/audioDeviceIndex the Settings → Audio combo uses; full config stays in Settings. MON button relabeled MON TX so its purpose is clear. ComboBox considered + rejected (long device names blow out the row width).

### New items logged (not started)

#165 (⚠ bug): AM/DSB modes only produce the UPPER sideband (lower

missing). AM/DSB are double-sideband → the WDSP passband for those modes is likely set one-sided (USB-only) instead of symmetric −W..+W. RX-DSP investigation, queued next.

GitHub / `main` branch: operator has questions to raise on return.

### Docs

USER_GUIDE updated for the rack dim/recolor, the monitor (MON TX + the

VAC/TCI routes), and the Out picker.

This session-log entry + a THETIS_DIRECT_PORT_PLAN.md reconciliation

pass (mark done-but-unmarked items, add a post-P4 shipped-work entry). .pdf/.docx of both NOT regenerated (no pandoc/doc-gen tool on this box) — MD sources updated; PDF/docx regen pending the operator's tool.

## 2026-06-16 (design + investigation day — NO commits; two items parked for tomorrow)

Context: native TX DSP rack (EQ/Speech/Combinator/Plate) + #49 Profile Manager bundle in flight; operator field-testing profiles on HL2+.

### #90 TX audio monitor — DESIGN LOCKED, parked for tomorrow's build

Operator ask: MON button + Monitor volume slider on the Audio panel upper row. Studied Thetis MON / MonitorVolume (gates processed TX audio into the RX audio mixer + SetIVACmon; the verbatim xMixAudio(0,0,…,out[2]) "mix monitor audio" line is already carried at CMaster.cpp:458, fed empty out[2]). Operator chose both: HL2 jack now, separate PC device later.

Key finding: WdspEngine::dispatchAudioFrame() (wdsp_engine.cpp:3100)

STAYS LIVE during TX — RX audio is just gain-zeroed by the txMuted_ && autoMuteOnTx_ gate, then OutBound(0) → EP2 LR → AK4951 jack. So the jack is live-but-silent during TX = the inject seam (no fight with RX stand-down).

Locked design — ONE shared tap: in xcmaster case 1 capture

pcm->in[stream] AFTER the rack, BEFORE fexchange0 (the processed "what you sound like" mic audio) → small SPSC ring.

Route 1 (HL2 jack, NOW): drain ring in dispatchAudioFrame; when MOX

up + MON on, output monitor audio (scaled by new Monitor-vol) in place of muted RX audio → existing OutBound(0) path. No new stream.

Route 2 (separate PC device, LATER): feed same tap into

xvacOUT(kVac1Id, stream 2) (replace vacMonSilence_) + SetIVACmon = IVAC Stage 5 / DL-4 done reference-faithfully.

Tradeoff (note in tooltip): post-rack tap is pre-WDSP-ALC/bandpass —

hear your rack DSP faithfully, not WDSP's corrective ALC.

Operator: "hold for tomorrow's build."

### #162 Speech-panel profile not saving — ROOT-CAUSED, parked first-thing tomorrow

Operator: TX Speech panel options aren't saved/recalled by Profiles while EQ/Combinator/Plate are. Decisive repro: turned De-esser ON, recalled an all-off profile → De-esser button stayed ON.

Traced the full path: capture (main.cpp:752) / apply (main.cpp:798) /

Profile toJson+fromJson+sameValues / SpeechModel saveState+loadState / single instance / context property — all symmetric & correct. The data DOES save and DOES apply to the model + DSP.

Root cause = QML binding-break (UI-refresh, not persistence):

SpeechPanel (and PlatePanel/CombinatorPanel) use Button{checkable:true; checked:<model>} + Slider{value:<model>}. The first click/drag makes Qt Quick write checked/value imperatively, severing the binding to the model. Recall then updates model+DSP+numeric labels but NOT the toggle/slider visuals — the button is a dead light. EqPanel dodges this (non-checkable toggles + pure checked: binding off a rev counter + Q_INVOKABLE getters). De-esser is the clearest demonstrator because a stuck binary toggle is unambiguous; Plate/Combinator share the latent pattern but only surface after mid-session interact-then-recall.

Fix (#162, high pri): adopt EqPanel's binding-safe pattern in the

three rack panels. QML-only, no DSP/wire risk.

### Tomorrow's order (operator-set)

#162 Speech (+Plate/Combinator) panel binding-safe rebuild — FIRST.

#90 TX monitor build (Route 1 HL2 jack; Route 2 PC device as the

follow-on within #90 / IVAC Stage 5).

Memory: project-lyra-cpp-monitor (the #90 locked design).

## 2026-06-13 (Saturday — ⭐ TX BRING-UP COMPLETE + v0.2.3 RELEASED)

`main` == `tx-rebuild` == HEAD `e8aafbc` (pushed).  Tags: milestone tx-working-2026-06-13 (at bd61d07) + release v0.2.3 (at a5dc763). GitHub Release v0.2.3 — "Transmit: SSB + digital (TCI) on the air" PUBLISHED with Lyra-Setup-0.2.3.exe (67.7 MB) attached, marked Latest.

### P4.b Wire-LIVE switchover — SHIPPED + operator HL2 bench PASSED

fb9ec41 P4.b-1 — Wire-LIVE RX-out gate (operator bench PASSED).

0b551b4 P4.b-2 — TX wire-LIVE: first modulated RF through the

direct-port chain (operator HL2 bench).

9db2c50 P4.b-2.1 — TX SSB sideband fix: USB transmits USB (9100-verified).

84dbb12 fix(tx): TUN panadapter spike on-marker — TX-analyzer crop

shifted by NCO−dial offset.  Verified Lyra TUN is Thetis-exact zero-beat (two-offset scheme; the bug was display-only — authoritative refs: console.cs:32553-32587 UpdateTXDDSFreq path, postgen 30788-30800, readout 22035-22051).

§7 dead-code retirements (3 build-gated commits): fe6a438

OutboundRing + Ep2SendThread, 99fd24a txWorkerLoop + composeCC + buildEp2KeepaliveTemplate, e711256 old HL2-audio-jack EP2 ring (pushAudio + setHl2AudioSink).

P4.b bench gates PASSED: SSB voice both sidebands + Phase-3-EXIT

RF-safety kill-test (RFG drops on hard-kill).

### TCI digital TX re-home — SHIPPED + ON-AIR VALIDATED

bd61d07 — filled the §10.3 src/tci/TciTxBridge.{h,cpp} skeleton

(singleton, mono deque, I=Q=mono drain w/ zero-fill underrun, queuedSamples() for CHRONO).  Registered via SendpInboundTCITxAudio(&TciTxBridge::inboundCb) after create_cmaster; fed by tci_server.cpp handleBinaryFrame tap; gated by SetTXTCIAudio(0, micSource=="tci").  The verbatim DSP seam (xcmaster case 1 use_tci_audio gate) was already present — only the host-side sink was missing.

Bench: MSHV → TCI → FT8 real on-air QSOs — N3YDN (FM29),

F4FLF (JN18, ~4160 mi DX), KO4OIG (EL89), W3EWL/QRP.

### Space-bar PTT toggle (#157) — SHIPPED

2824ebf — Prefs.spaceBarPttEnabled (tx/space_bar_ptt_enabled,

default ON), MainWindow keydown/keyup gate, Settings → Hardware → Transmit checkbox under the HW-PTT tickbox.  Operator-verified working.

### Release

e8aafbc — version bump 0.2.2 → 0.2.3 (CMakeLists project(VERSION) +

installer/lyra.iss AppVersion).  Clean build, ISCC installer, tag, GitHub release (notes + installer).

### NEXT (morning)

Mic-input paths discussion (PC/VAC host audio + plain-HL2 vs HL2+).

Key reframe: a plain HL2 (no AK4951) has no radio-side mic at all → host audio-in (#102 VAC1) is REQUIRED for those ops to TX phone, not optional.  All sources are interchangeable 48 k mono feeders of the same InboundTCITxAudio seam (TciTxBridge = the template).  §5.4 "single mic-source class, no abstraction" rule has expired now that there are 4 concretes — introduce ITxAudioSource.  Capability-aware default (plain HL2 → PC/VAC; HL2+ → AK4951 mic).  Decide: TX-in-only vs full bidirectional VAC; Qt Multimedia vs WDSP rmatchV.

Parked: #156 restart-after-hard-kill-mid-TX hang; intermittent

menu-drag display stutter (monitoring; ruled out as a §7 regression);

#104 HL2 Line In (codec mux); #103 VAC2.

## 2026-06-12 (Friday PM-4 — P4.a prep SHIPPED, both commits wire-inert)

Branch: tx-rebuild, HEAD baef866 (pushed).

### P4.a-1 SHIPPED `a4aa8c3` — sendProtocol1Samples verbatim (DORMANT)

NEW src/wire/NetworkProto1.cpp — networkproto1.c PARTIAL: the EP2 writer thread sendProtocol1Samples verbatim (networkproto1.c:1204-1267).  Mechanical diff IDENTICAL (whitespace-normalized, accommodations mapped back); sole structural delta = braces around the DEFERRED non-HL2 else-branch (comment-only branch needs braces to compile).  Accommodations documented in the preamble: lyra::wire namespace; the HPSDRModel enum-class mapping; write_main_loop_hl2(prn->OutBufp) = the FrameComposer monolithic WriteMainLoop_HL2 equivalent (#121/#122 fold); generic WriteMainLoop (non-HL2/ANAN P1) carried as DEFERRED reference text; (AVRT_PRIORITY) cast + suppress 4100 on the verbatim signature.

io_keep_running global added (network.h:411 verbatim; init 0 so the loop body cannot run even if started prematurely) + the decl in RadioNet.h.

FrameComposer write_main_loop_hl2 tail flipped from the Step-14 outbound_notify_consumed_pair() translation to the verbatim ReleaseSemaphore(prn->hobbuffsRun[0/1], 1, 0) pair.  Dormant-safe: nothing calls write_main_loop_hl2 until P4.b starts the writer; the hobbuffsRun handles are created at the P4.b open() with the quartet.

DORMANT: nothing _beginthreadex's the function until P4.b's open() (reference start site = StartAudio, netInterface.c:66 + the quartet :68-71).  Build clean, zero new warnings.

### P4.a-2 SHIPPED `baef866` — PostGen seam entries + verbatim field types

wdspcalls: SetTXAPostGen{Run,Mode,ToneMag,ToneFreq,TTMag,TTFreq} for the P4.b TUN re-home (legacy DC-injection TUN dies with the legacy EP2 packer; TUN becomes the reference TXA output-side tone generator).  Pre-cdef audit: NO header declares these — PORT-exported definition sites in wdsp/gen.c only (gen.c:784/792/800/808/817/826), signatures harvested from the definition bodies; presence + exact casing verified against the bundled wdsp.dll export table (PE scan, all 6 PRESENT of 533).  TT pair included per table rule #3 (PS committed; the PS calibration drive is the two-tone).

TxReadBufp: std::vector -> verbatim double* (network.h:62) + the verbatim calloc at create_rnet (netInterface.c:1604, 2*sizeof(double)*720, process lifetime — the P2.b OutBufp precedent).  Ep6RecvThread's 5 .data() sites -> bare pointer; the vector-size guard reduces to a null check (fixed 1440-double capacity >= 4*spr for every valid nddc).

hWriteThreadMain: std::thread -> verbatim HANDLE (network.h:90).  Zero users existed on the std::thread form; P4.b assigns the verbatim _beginthreadex(sendProtocol1Samples) handle.

Build clean (sole warning = the pre-existing hl2_stream C4456, untouched file).  Commit-message nit: a backtick-quoted literal got eaten by bash substitution again (the P2.b class) — citation survives, no force-push.

### NEXT = P4.b (THE switchover — ONE commit, full operator HL2 bench gate)

open(): quartet CreateSemaphore ×4 + prn->hWriteThreadMain = _beginthreadex(sendProtocol1Samples) (io_keep_running=1 BEFORE start).

dispatchAudioFrame -> the asioOUT-pattern tee (HL2-jack = real audio into OutBound(0); PC = zeroed tee so EP2 pacing never starves) — the B.6.b-delicate RX switchover.

Inbound(inid(1,0), mic_sample_count, prn->TxReadBufp) at the live EP6 mic-harvest site (Ep6RecvThread, the block already mirrors networkproto1.c:560-579).

FSM re-home: TXA SetChannelState arming per the §15.25 ground truth + TUN -> SetTXAPostGen.

Control-plane mapping per the design doc §5 table; retirements per §7 (txWorkerLoop + keepalive + DC-TUN + slew-fill, OutboundRing, Ep2SendThread, FrameComposer cv tail).

Teardown: io_keep_running=0 -> release both hsend sems once -> join -> CloseHandle quartet -> destroy_obbuffs BEFORE destroy_xmtr.

Bench gate per design doc §8 incl. explicit TX-state lines + FIRST VOICE through the chain.

## 2026-06-12 (Friday PM-3 — P2 audit complete + P2.a eer completion SHIPPED)

Branch: tx-rebuild, on top of 2897756.

### P2 fidelity audit (the deliverable that scopes P2.b/c/P4)

Reference TX-out chain (HL2 P1), fully read: ob_main -> sendOutbound (network.c:1237; P1 branch :1285-1340) — id 1 memcpys 126 complex into prn->outIQbufp + Release(hsendIQSem) + Wait(hobbuffsRun[0]); id 0 -> outLRbufp + hsendLRSem + hobbuffsRun[1] -> sendProtocol1Samples thread (networkproto1.c:1204-1267): WaitForMultipleObjects(both, TRUE) -> eer overwrite if peer->run && XmitBit -> !XmitBit => memset outIQbufp -> swap_audio_channels -> 16-bit BE quantize + HL2 CW-bit overlay into OutBufp -> WriteMainLoop_HL2 -> MetisWriteFrame -> Release both hobbuffsRun.  Resources: bufs calloc'd netInterface.c:1606-1608; semaphore quartet created in StartAudio :68-71; obbuffs rings 0/1 created in UpdateRadioProtocolSampleSize :1856-1857; eer created run=0 per xmtr (cmaster.c:212-224).

Lyra dormant Step-14 surface: functionally-parallel idiom translations predating the verbatim mandate — prn buffers are std::vector (RadioNet.h:552-554), the semaphore quartet became OutboundRing's bool+cv, Ep2SendThread is the sendProtocol1Samples rewrite.  XmitBit is already verbatim (extern int, RadioNet.h:733).  Disposition: re-port verbatim, retire the translations at P4 Wire-LIVE.

Landmine found: BOTH reference functions deref pcm->xmtr[0].peer->run, but P0.d had deferred the eer creation — peer was NULL.  That made P2.a unambiguous and first.

### P2.a SHIPPED — eer completion (verbatim)

wire/cmcomm.h: opaque eer typedef replaced by the FULL verbatim struct (wdsp/eer.h:30-49, mechanical diff IDENTICAL); opaque DELAY twin typedef added (wdsp/delay.h:32-59).

wire/wdspcalls.{h,cpp}: +5 seam entries — create_eer / destroy_eer / xeer / pSetEERSize / pSetEERSamplerate (signatures verbatim from eer.h:51-75; all five verified present in the bundled wdsp.dll via PE export-table scan).

wire/CMaster.cpp: all four deferred eer sites RESTORED verbatim — create_xmtr create_eer(run=0, ...) 12-arg block (cmaster.c:212-224); destroy_xmtr destroy_eer (:262, after destroy_ilv per reference order); xcmaster xeer(pcm->xmtr[tx].peer) (no-op at run=0 with in==out — verified in eer.c:86-122); SetXmtrChannelOutrate pSetEERSamplerate/pSetEERSize.  Restored-lines verbatim-subset check PASS (18 lines).

Runtime impact: create_xmtr now also creates/destroys one run=0 eer object per xmtr (heap + CS only); xeer in the TX pump body never executes today (stream-1 pump has no producer).  RX path untouched.  Clean build, zero warnings.

### P2.b SHIPPED (same session) — prn outbound surface verbatim

outLRbufp/outIQbufp/OutBufp std::vector -> the VERBATIM reference pointer fields (network.h:64-66) + the verbatim calloc set at create_rnet (netInterface.c:1606-1608, mechanical subset PASS).  Lifetime = process (create_rnet call-once) = reference.

Verbatim HANDLE quartet declared on prn (hsendLRSem/hsendIQSem/hsendEventHandles[2]/hobbuffsRun[2], network.h:92-95) — DORMANT/nullptr until P4's StartAudio equivalent; the Step-14 cv+flags translation stays alongside until P4 retires its consumers.

Callers bent: OutboundRing .data() ×6 -> raw; Ep2SendThread pack casts -> char; write_main_loop_hl2 parameter -> const char* (reference type).  Wire-inert (only create_rnet's allocation runs live).  The lone C4456 in hl2_stream.cpp:776 is PRE-EXISTING (file untouched; surfaced by the header-triggered recompile) — flagged for a separate cleanup.

### P2.c SHIPPED (same session) — sendOutbound verbatim + ob_main restore

NEW src/wire/Network.cpp — network.c PARTIAL direct port: sendOutbound verbatim (network.c:1237-1341).  P1 branch LIVE (the outLRbufp/outIQbufp memcpy + hsendLRSem/hsendIQSem release + hobbuffsRun wait handshake, incl. the EER de-interleave sub-branch with its function-local static ptr); ETH branch carried as DEFERRED reference text (Protocol 2/ANAN = v0.4; WriteUDPFrame/udpOUT unported).  Accommodations, each documented inline: the RadioNet.h enum-class mapping on the protocol gate; suppress 4101 (ETH-only locals) + 4456 (the reference's own loop-scope i shadowing its function-scope i, network.c:1239 vs :1298).  Mechanical diff vs network.c:1237-1341 = IDENTICAL (whitespace-normalized, deferred text restored).

wire/ObBuffs.cpp ob_main: the DEFERRED sendOutbound(id, a->out) hand-off RESTORED verbatim — the P1 TU is now reference-complete.

Still dormant end-to-end: no create_obbuffs caller until P3/P4; the semaphore quartet is created by P4's StartAudio equivalent before the pump can deliver, the same ordering the reference relies on.  Clean build, zero warnings.

### P3 SHIPPED (same session; survived + re-verified after an operator power outage mid-arc) — netInterface registrations + obbuffs ring lifecycle

create_rnet moved to once-per-process at the main.cpp QTimer block AFTER create_xmtr — the reference's C#-init ordering (create_cmaster -> create_xmtr -> create_rnet -> StartAudio).  The old per-open call re-allocated prn on EVERY stop/start (leak + wire-state reset, contradicting the close() "prn stays alive for re-open" contract — a latent pre-existing defect the P3 reference-read surfaced).  open() now asserts the reference prn-non-null contract (netInterface.c:40); if wdsp.dll failed to load, open() refuses with a qCritical (no WDSP = no radio, the reference posture).

SendpOutboundTx(OutBound) restored at the reference site (netInterface.c:1761, tail of create_rnet) — pcm->OutboundTx -> xmtr[0].pilv via the verbatim no-guard SetILVOutputPointer; ordering safe by construction.  The Stage C.3-fix "registration belongs at the xmtr-open site" note SUPERSEDED (it guarded a Step-14 stub against per-open clobber; under the direct port the reference site is correct).  RX side deliberately NOT registered — WdspEngine's openRx1 owns RX dispatch in the approved hybrid until P4 (registering RX here = the B.6.b clobber = dead RX audio).

UpdateRadioProtocolSampleSize ported verbatim (netInterface.c:1836-1858, mechanical diff IDENTICAL with the documented kMax*/enum-class accommodations) — called per session-open from HL2Stream::open() at the StartAudio position (:45): per-protocol spp values (USB: mic 63 / rx 63 / tx 126 / audio 126) + create_obbuffs(0/1) — TWO new ob_main pump threads per session, both idle (ring 0 has no producer until P4's RX switchover; ring 1's producer is xilv in the stream-1 cm pump, quiescent until P4 feeds Inbound(1,...)).  destroy_obbuffs(0/1) at close() per StopAudio (:112-113).

Power-outage recheck: all 4 edited files probe-verified intact, diff-vs-HEAD = exactly the P3 hunks, forced rebuild of the touched TUs clean (sole warning = the pre-existing hl2_stream C4456), mechanical diffs re-PASSED post-outage.

[DONE] OPERATOR HL2 BENCH PASSED (same day): "rx fine" — RX working on the P3 lifecycle (once-per-process create_rnet, per-session obbuffs rings + 2 idle pump threads, TX registration live).  P3 gate cleared; P4 Wire-LIVE unblocked.

### ⚠ Operator-flagged near-miss + bench-discipline correction (2026-06-12)

Operator accidentally pressed TUN a few sessions back and got REAL RF OUT, despite the rebuild-arc bench notes repeatedly saying "TX stays wire-quiescent / RX-only."  Code-verified: those statements were true of the NEW direct-port chain only.  The OLD hardware-validated TX-0c path — TUN DC-injection at the EP2 packer (hl2_stream.cpp:~2516) + the requestMox FSM + ATT-on-TX — was NEVER part of the TX rip (only the SSB voice DSP subsystem was removed) and has been LIVE on tx-rebuild the whole time.  RF required the operator's own persisted PA-enable opt-in + drive settings (the deliberate-arm gate worked as designed; it was armed from the first-RF benches and persisted).

Operator decision: LEAVE IT — it is the proven path, gated behind the PA-enable opt-in; P4 replaces it under its own full bench.  (Disarm anytime: un-tick Enable PA in Settings.)

Discipline correction going forward: every direct-port bench gate now includes an explicit "TX state unchanged" line (TUN/MOX behavior + PA-enable posture), and "RX-only / wire-quiescent" is reserved for statements about the WHOLE wire, not just the new chain.  Applies to the P4 gate and onward.

### P2 COMPLETE.  NEXT

P3 — netInterface outbound registrations: SendpOutboundRx(OutBound) / SendpOutboundTx(OutBound) per netInterface.c:1749-1761 — MUST register AFTER create_xmtr (verbatim setters have NO null guards); plus the create_obbuffs(0/1) sites (netInterface.c:1856-1857) at the Lyra equivalent of UpdateRadioProtocolSampleSize.

P4 — Wire-LIVE switchover (one commit, full HL2 bench gate): verbatim sendProtocol1Samples thread + the semaphore-quartet creation + WriteMainLoop_HL2 hand-off; retires the OutboundRing/Ep2SendThread cv translations.

## 2026-06-12 (Friday PM — P1 obbuffs.c verbatim direct port SHIPPED)

Branch: tx-rebuild, on top of 976062d (the P0.d bench-PASS doc flip).

### Shipped (one commit)

NEW `src/wire/ObBuffs.{h,cpp}` — reference obbuffs.{h,c} verbatim: obb,*OBB twin typedef + numRings/obMAXSIZE/OBB_MULT defines + the file-scope _obpointers obp four-alias bank + create/destroy/flush/OutBound/obdata/ob_main/SetOBRingOutsize.  The TX-OUT seam (WDSP output → ring → ob_main pump thread → sendOutbound → protocol packers); SEPARATE TU from cmbuffs (inbound-only).

The reference's own 2014-era idioms kept as shipped (deliberately NOT harmonized to the cmbuffs sibling): calloc/free (not malloc0/_aligned_free), destroy-time obp.pcbuff[0]==NULL guard, obdata WITHOUT the MW0LGE CS wrap, the csOUT enter/leave pair at the top of the ob_main loop, the out work buffer inside the struct.

Sole deferred line: sendOutbound(id, a->out); in ob_main — the reference defines it at network.c:1237 (the P1 branch :1285-1340 is the outIQbufp/outLRbufp + hsendIQSem/hsendLRSem/hobbuffsRun handshake WriteMainLoop_HL2 consumes) = P2 scope.  Carried in place as reference text with a DEFERRED tag; decl in ObBuffs.h verbatim.

Provenance check resolved before code: the "OutBound at network.c:1285-1340" citation in older RadioNet.cpp comments is the PROTOCOL_1 branch of sendOutbound, not a second OutBound — obbuffs.c's OutBound is the one registered via SendpOutboundRx/Tx (P3).

Compiler accommodations: same set as CmBuffs.cpp (suppress 4312/4189/4311/4302 on the thread-arg casts + (AVRT_PRIORITY) 2), each marked inline.

### Verification

Clean build, ZERO warnings.

Mechanical diff vs reference: ObBuffs.cpp IDENTICAL (code lines, whitespace-normalized); ObBuffs.h sole delta = the reference's include-guard #define _obbuffs_h, replaced by #pragma once (documented packaging difference, same as CmBuffs.h).  Live-line-subset PASS on both.

### Wire impact

NONE — the TU is dormant.  No Lyra caller of create_obbuffs exists until P3/P4 (the reference creates rings 0/1 in netInterface.c:1856-1857).  No new threads at startup, wire bytes unchanged.  An operator RX smoke at convenience is prudent (binary changed) but this touches no live path.

### NEXT

P2 = sendOutbound / sendProtocol1Samples fidelity audit of the dormant wire layer (reconcile network.c:1237-1341 + networkproto1.c sendProtocol1Samples vs Lyra's Step-14-era OutboundRing/MetisFrame/Ep2SendThread surface), then restore the ob_main hand-off verbatim.

P3 netInterface registration (SendpOutboundRx/Tx(OutBound) — AFTER create_xmtr, the verbatim setters have NO null guards) → P4 Wire-LIVE (one commit, full HL2 bench gate).

## 2026-06-12 (Friday — P0.d CmBuffs/CMaster/cmsetup verbatim direct port SHIPPED + ✅ HL2 RX-regression bench PASSED)

Branch: tx-rebuild HEAD = afc7950, pushed to origin/tx-rebuild.

✅ OPERATOR HL2 BENCH (same day): RX-regression gate PASSED ("RX still working") — clean RX on the new per-stream cmbuffs pump/ring layout (two cm_main threads, stream 0 idle by design).  P0.d is fully closed; P1 (obbuffs.c verbatim port) is UNBLOCKED and starts next session.

### Shipped (one commit, `afc7950`)

NEW `src/wire/cmsetup.{h,cpp}` — reference cmsetup.{h,c} verbatim: cmMAX* sizing macros (16/4/4/2/32), rxid/txid/sp0id/stype/chid/inid/mixinid/getbuffsize, SetRadioStructure + set_cmdefault_rates.  CreateRadio/DestroyRadio carried with the unported pipe/sync calls commented.

`src/wire/CmBuffs.{h,cpp}` rewritten verbatim — cmb,*CMB twin typedef, #define CMB_MULT (3), malloc0/_aligned_free, no calloc/intptr_t/guard deviations; pcm->in[] allocation moved back to create_cmaster (reference shape).

`src/wire/CMaster.{h,cpp}` rewritten verbatim — FULL _cmaster struct (cmaster.h:39-99, PS surface incl. out[3]/panalalloc/pgain/peer), cmaster,*CMASTER, raw TCI fn ptrs, enum AudioCODEC, cm = {0}.  TxChannel RAII carve-out DELETED (src/wdsp/TxChannel.{h,cpp} retired): verbatim no-arg create_xmtr opens the WDSP TXA channel (chid(1,0)=1) + TX analyzer (disp 1; pan analyzer = disp 0, no collision) + out[0..2] + create_ilv itself through the wdspcalls seam.  create_cmaster/destroy_cmaster verbatim per-stream loops (update[] CS + cmbuffs + in[] for all cmSTREAM streams; create_rcvr = deferred stub, RX hybrid).  xcmaster verbatim: update[] critical section restored, real stype/txid/chid, TCI-override memset restored; fexchange0 + monitor-mix xMixAudio (accept-gated, quiescent vs the 1-input WdspEngine mixer) + xilv live.  SetXcmInrate/SetCMAudioOutrate/SetRcvr-/SetXmtrChannelOutrate/SetRunPanadapter/SetAntiVOXSource* ported.  All deferred subsystem lines carried IN PLACE as reference text with DEFERRED tags.

`src/wire/cmcomm.h` — opaque verbatim twin typedefs ANB/NOB/EER/VOX/TXGAIN/ANALYZERS (tags match reference headers; completing a type later is source-compatible) + the umbrella-mapping note (.cpp files include explicit family headers — an include-list umbrella would be circular under #pragma once because the family headers include cmcomm.h for the base surface).

main.cpp — SetRadioStructure(2,1,1,1,0,…)/set_cmdefault_rates(48 k) config block BEFORE create_cmaster (derived ids match the live layout: chid(0,0)=0 = WdspEngine RX1 channel, chid(1,0)=1 = TX channel); create_xmtr() invoked in the QTimer block after resolve_wdsp_calls (the documented DEFERRED-CALLSITE accommodation); handler-1.5 = gated destroy_xmtr().  Consumer-side decls for the headerless PORT functions (test_ilv precedent).  RadioNet.cpp → enum AudioCODEC cases; scratch/test_ilv.cpp → verbatim cmaster cm = {0}; globals.

### Verification

Clean build, ZERO warnings (C4701 in getChannelOutputRate disabled function-wide with a documented verbatim-text-wins pragma — line-level suppress can't reach the code-gen-stage warning).

scratch/test_ilv.exe ALL PASS against the new verbatim globals.

Mechanical diff vs reference: CmBuffs.{h,cpp} / cmsetup.h / CMaster.h struct+decls+enum / all 10 fully-verbatim cmaster.c functions IDENTICAL (whitespace-normalized); cmsetup.cpp comment-only deltas; live-line-subset check PASS for the partially-deferred bodies.  Sole code accommodation: (char *)"" at the XCreateAnalyzer call (C++ string-literal constness).

### Behavior changes at startup (for the bench)

TWO cm_main pump threads now start (streams 0 + 1; stream 0 idles forever — no Inbound producer; the reference layout).  Stream rings sized from cmMAXInRate=384000 (in[] = 512 complex).

TX stays wire-quiescent: no Inbound() producer yet, pcm->OutboundTx null until P3.

destroy ordering: handler-1.5 destroy_xmtr() (gated) → handler-4 destroy_cmaster() = reference relative order.

### NEXT

~~Operator HL2 RX-regression bench (the P0.d gate)~~ — ✅ PASSED 2026-06-12 (RX working; gate cleared).

P1 = obbuffs.c port (TX-out seam, separate TU from cmbuffs) → P2 sendOutbound audit → P3 netInterface registration (outbounds AFTER create_xmtr) → P4 Wire-LIVE (one commit, HL2 bench gate).

## 2026-06-08 (Monday EOD — STAGE B aamix.c port COMPLETE + bench-validated)

Branch: tx-rebuild HEAD = 533b06b. Pushed to origin/tx-rebuild (0/0 in sync). origin/main deliberately untouched (different arc). Backup: _backups/lyra-cpp-2026-06-08-aamix-stage-B-COMPLETE.bundle (16 MB, git bundle --all).

### Today's shipped commits (in order, all on `tx-rebuild`)

533b06b [aamix port -- Stage B.6.b-final]   strip diagnostic instrumentation
8b8e0da [aamix port -- Stage B.6.b-fix1]    remove Stage-A NO-OP SendpOutboundRx stub (THE FIX)
12369bc [aamix port -- Stage B.6.b-debug-sink] dispatchAudioFrame sink-side counters
225518c [aamix port -- Stage B.6.b-debug]   9-counter chain instrumentation
53d9e5a [aamix port -- Stage B.6.b-retry]   wire AAMix via reference-faithful path
6e773a5 Revert "[aamix port -- Stage B.6.b] wire AAMix(1-input passthrough) into RX path"
525c2f0 [aamix port -- Stage B.6.b]         wire AAMix(1-input passthrough) -- REVERTED at bench
27ac2fa [aamix port -- Stage B.6.a]         extract feedIq audio tail into dispatchAudioFrame

### Arc summary

B.0 → B.5: AAMix.h/cpp port complete from earlier sessions (538-line header verbatim from aamix.c with GPL v3+ attribution to NR0V/WDSP, 1100+ lines of cpp; idiom translations from Win32 to C++23: CRITICAL_SECTION→std::mutex, HANDLE semaphore→std::counting_semaphore, _beginthread→std::jthread, _Interlocked*→std::atomic with fetch_or/fetch_and, malloc0→std::vector RAII).

B.6.a (`27ac2fa`): Refactored feedIq audio tail (lines 2402-2456) into new WdspEngine::dispatchAudioFrame(audio, nframes) helper. Byte-identical relocation; bench-confirmed.

B.6.b first attempt (`525c2f0`): Wired aaMix_ into RX path with create_aamix(active=0x01L) shortcut. Operator bench: NO audio. Reverted (6e773a5).

B.6.b-retry (`53d9e5a`): Reference-faithful per cmaster.c:297-313: create_aamix(active=0L, ring_size=4096, slew=0/10/0/10ms) → SetAAudioMixOutputPointer(aaMix_, 0, outbound) → SetAAudioMixState(aaMix_, 0, 0, 1) (activate via close_mixer/open_mixer slew atom). Operator bench: STILL NO audio.

B.6.b-debug (`225518c`): Chain-side instrumentation — 9 atomic counters + 1 ENTRY beacon + producer/consumer 1-Hz log emits in xMixAudio / mix_main. Operator bench result: chain healthy end-to-end (xmix=rel=wake=out=188/sec, nzOut=99%, mixmain_started=1, Outbound=set) but operator confirmed STILL no audio. ⇒ defect is downstream of Outbound.

B.6.b-debug-sink (`12369bc`): Dispatch-side instrumentation — 6 atomic counters in dispatchAudioFrame + 1-Hz log line tag [disp-dbg]. Operator bench result: [disp-dbg] lines NEVER fired in the entire 33-second capture. ⇒ dispatchAudioFrame is never entered. ⇒ the Outbound callable AAMix is calling is NOT the lambda WdspEngine wired.

B.6.b-fix1 (`8b8e0da`, THE FIX): Root cause traced from bench timestamps:

19:22:02.972 — WdspEngine::openRx1 calls create_aamix(id=0, ..., Outbound=dispatchAudioFrame_lambda) ⇒ paamix[0]->Outbound = real lambda ✓

19:22:02.973 — HL2Stream::open() → create_rnet() → SendpOutboundRx(STUB) ⇒ pcm->OutboundRx = STUB, then SetAAudioMixOutputPointer(nullptr, 0, STUB) resolves nullptr → paamix[0] ⇒ `aaMix_->Outbound = STUB`, real lambda CLOBBERED ✗

The Stage-A NO-OP stub at RadioNet.cpp:272-283 was a placeholder ("Stage B aamix port wires it via SetAAudioMixOutputPointer") for the wire-up Stage B was supposed to replace. Stage B.6.b correctly wired dispatchAudioFrame in openRx1 — but left the Stage-A stub registration in place, which then overwrote the wiring 1 ms later when HL2Stream::open() called create_rnet().

Fix: delete the Stage-A stub registration. Comment block on the deleted case warns future hands that the wire-up needs the per-channel sink-routing context (hl2Out_, hl2AudioPush_, audioRing_) that the global pcm cannot reach — so the wire-up must live at openRx1, not create_rnet.

B.6.b-final (`533b06b`): Strip diagnostic scaffolding. -134 lines. Brief comment in AAMix.cpp points future hands at commit history (git show 225518c for chain instrument, git show 12369bc for dispatch instrument).

### Operator HL2+ bench (final, 19:27:49 → 19:28:47 — 58-second run)

HL2 jack startup: first [disp-dbg] post-unmute: calls=188 muted=0 gain=0.0845 peak16=32767 hl2(push/null)=188/0 hl2pushSet=1 audioRingSet=0 — full-scale audio reaching AK4951.

Operator-confirmed audible RX ("Yes we have audio now").

Mute toggle via dispatch (muted counter 0→22→133→287).

Output device switched to PC Soundcard at 19:28:17 → hl2Out=0, audioRingSet=1, pc(push) 176→928/sec.

Switched back to HL2 jack at 19:28:23 → hl2Out=1, hl2(push) resumed climbing.

AGC mode changes (med/slow/off/fast) clean.

Chain out=N tracks 1:1 with dispatch calls=N across both sinks across the entire run — no dropped frames, no thread starvation.

### Tasks closed

#130 [completed] Stage B.6 [DEFER] — migrate RX audio path to ported AAMix

#132 [completed] Stage B.6.b — wire AAMix(1-input passthrough) into RX path

Stages B.0–B.5 + B.6.a already closed in earlier sessions.

### Methodology lessons recorded (for future Stage X work)

2-stage instrumentation (chain + dispatch) localized the defect to a single line of code on the SECOND bench. No speculation cycle, no convergence theatre.

Operator-empirical rule held: after first B.6.b failure I was tempted to revert the AAMix port itself; the chain counters proved AAMix was correct in isolation, redirecting the dig to the wire-up surface where the bug actually was.

Hard rule for next port: counters first when the symptom is "nothing happens". Instrumentation cost is trivial vs the alternative (multi-round revert/speculate cycle).

"Do as Thetis does, Lyra-Native style" continues to hold — the first B.6.b attempt's active=0x01 shortcut cost a revert cycle; the reference's active=0 then SetAAudioMixState(activate) pattern worked first time on the retry.

### Memory + doc updates

MEMORY.md index line for lyra-cpp-tx rewritten with "READ FIRST 2026-06-09 AM" header + branch/HEAD/backup/root-cause pointer.

project_lyra_cpp_tx.md — new "▶ READ FIRST 2026-06-09 AM resume pointer" block at top, plus full "State (2026-06-08 EOD)" section. Earlier state preserved below.

EXECUTION_PLAN.md — new "PROGRESS TRACKING — STAGE B (aamix.c port) [SIDE-TRACK]" section with full B.0 → B.6.b-final checklist; new row in VERIFICATION LOG; footer "Status as of 2026-06-08 EOD" updated.

This SESSION_LOG entry.

PDF/DOCX regenerated via tools/sync_execution_plan.py.

### Next session pointer

First action: git fetch origin THEN git status + git log --oneline -5 tx-rebuild origin/main to surface divergence.

Next port target = OPERATOR'S CALL. Stage B side-track done; candidates:

Stage C: ilv.c port (TX I/Q interleaver — pairs with the SendpOutboundTx stub already wired in CMaster.cpp Stage A)

Stage D: xcmaster pump body port

Or pivot back to Step 14 / Phase 3 wire-layer rebuild on main (previous-arc resume was TX-1 Component 7 — first end-to-end SSB voice TX)

### Wire status

RX audio flows live through the ported AAMix dispatcher on the tx-rebuild branch's HL2Stream::open() path. TX path unchanged from earlier tx-rebuild state.

## 2026-06-06 (Saturday LATE-AFTERNOON — §1-C FULLY COMPLETE + RE-AUDIT CLEAN)

Branch: tx-rebuild HEAD = 169f1c2.  Build clean, lyra.exe relinked at 11:10:09.  16 commits shipped today + TWO full multi-agent audit rounds.  §1-C arc COMPLETE through Stage 4F.2 (FrameComposer dissolution).  Final re-audit returned 37/37 PASS across 3 sections, zero remaining §6-Q3/Q5-class candidates, zero forbidden tokens in src/wire/.

### What's reference-faithful now

TX wire layer: 100% reference-faithful (all _radionet

fields in RadioNet; all reference file-scope globals as TU-scope statics in correct .cpp; Router + OutboundRing + FrameComposer all dissolved into free functions matching reference; symmetric socket-binding via metis_wire_bind() in both Ep6RecvThread + Ep2SendThread).

§1.1 verdict: 🔴 → ✅ PARITY (Stage 4E doc consolidation).

### What's NOT yet reference-faithful (tracked, NOT in §1-C)

Task #73 — pre-existing forbidden-token violations

(Thetis / Console.cs / OpenHPSDR / PowerSDR) in 5 non-wire files: tci_server.{h,cpp}, mainwindow.cpp, hl2_stream.{h,cpp}, wdsp_native.h.  Doc-style cleanup, ~1 hour, no impact on shipped wire-layer paths.

Wire-INERT — nothing in HL2Stream::open() calls the

new wire-layer free functions yet (step 14 wire-up).

Task #114 deferrals — EER mode, non-HL2

WriteMainLoop_generic, PeakFwdPower/PeakRevPower helpers. Hardware-blocker (no ANAN hardware to bench-test).

### Step 14 scope (NOT trivial)

HL2Stream is 4203 lines total with its own monolithic rxWorkerLoop + txWorkerLoop member methods at hl2_stream.cpp:921 + :2437 (the OLD wire path).  Step 14 must REPLACE these (per Rule 7 "Delete TX, don't refactor") with calls into the new wire-layer free functions:

prn global pointer assigned at session-open

metis_wire_bind(socket, dest, dest_len)

outbound_init() to size prn->outLRbufp/outIQbufp

ForceCandC::prime(3, tx_freq, rx_freq) synchronously

per §7 temporal-separation contract

Ep6RecvThread::start(socket_fd) → spins RX thread

reading from prn-> + WSAEventSelect

Ep2SendThread::start(socket, dest, dest_len) → spins

TX thread; no more composer / ring params (Stage 4F.2)

Migrate HL2Stream's existing telemetry/mic/IQ callbacks

to the new sink-based registration (Router::register_sink for IQ; Ep6RecvThread::set_*_sink for telemetry/mic/I2C)

Plan-first commit (R6).  Bench-critical (first time the new wire layer actually emits datagrams to the radio). Multi-hour, possibly multi-commit.  Wrong wire-up = broken RX/TX on real HL2 hardware.

### Resume after step 14

Operator-bench verification of RX-side audio + telemetry on real HL2+ hardware.  Then TX-side bench (CW/SSB + Palstar power readings).  Then Task #73 cleanup (small, doc-only, can happen anytime).

### Wire status

Wire-INERT.  All wire-layer components reference-faithful but not connected to runtime.

## 2026-06-06 (Saturday AFTERNOON — §1.1 REVERT COMPLETE — historical)

Branch: tx-rebuild HEAD = 79fe4ec.  Build clean, lyra.exe relinked at 10:45:10.  TWELVE commits shipped today + 6-agent comprehensive TX audit done.  ALL stages of the §1-C sweep complete in code (§1.1 networking- infrastructure exclusion fully reverted).  Only Stage 4E (PARITY_CHECKPOINTS doc consolidation) remains before step 14 wire-up.

### Today's commits (newest first)

79fe4ec §1-C Stage 4D — OutboundRing dissolves to free functions (§1.1 revert COMPLETE)

1d73bea §1-C Stage 4C — Ep2SendThread reads from prn->OutBufp + g_fpga_write_bufp

f1031da §1-C Stage 4B.1 — correct FPGAReadBufp scope (file-scope, not _radionet)

26acea3 §1-C Stage 4B — Ep6RecvThread reads from prn-> + WSAEventSelect

e157301 §1-C Stage 4A — add networking-infrastructure fields to RadioNet

d88ddd6 §1-C Stage 3 — OutboundRing wait-all via condition_variable

2b78d9e §1-C Stage 2A — Router class → struct + free functions

0e2a375 §1-C Stage 1 — small reverts (wb_enable, push timeout, diag counters)

50b713b §6-B nit — drop metis_write_frame null-guard

3bfd248 §6-B + §7 — TU-scope MetisFrame + ForceCandC populate

### §1.1 revert summary

All _radionet fields the original §1.1 exclusion (signed 🔴 2026-06-04) had punted to wire-layer class members now live in RadioNet as members exactly per reference:

Buffer pointers: RxBuff, TxReadBufp, outLRbufp,

outIQbufp, OutBufp (+ ReadBufp declared P2-future-use)

RX seq counters: cc_seq_no, cc_seq_err (P2-only;

declared in RadioNet but HL2-inert)

Thread handles: hReadThreadMain, hWriteThreadMain,

hKeepAliveThread (std::thread types)

Init semaphores: hReadThreadInitSem, hWriteThreadInitSem

(std::counting_semaphore<1>)

Outbound sync (cv-collapsed mirror of 4 reference HANDLE

pairs): cv_outbound + mu_outbound + 4 bool flags + outbound_stop

WSA event + waitable timer (Win32-only): hDataEvent,

wsaProcessEvents, hTimer, liDueTime

Networking config ports: p2_custom_port_base,

base_outbound_port

Reference-FILE-SCOPE globals (NOT in _radionet) live as TU-scope statics in their respective wire-layer .cpp files per the §6-B precedent:

wire/MetisFrame.cpp: g_metis_out_seq_num (was Ep2SendThread::out_seq_num_)

wire/Ep6RecvThread.cpp: g_metis_last_recv_seq, g_seq_error, g_seq_seen, g_control_bytes_in[5], g_fpga_read_bufp

wire/Ep2SendThread.cpp: g_fpga_write_bufp

Wire-layer classes dissolved or reduced:

Router: class → plain struct + free functions (Stage 2A)

OutboundRing: class → namespace-scope free functions

operating on prn->... (Stage 4D)

Ep6RecvThread: still a class (thread orchestrator), but

all data moved to prn->... or TU-scope statics

Ep2SendThread: same — thread orchestrator, data moved out

### Remaining — Stage 4E (doc-only)

Consolidate the §1-C entry in docs/architecture/PARITY_CHECKPOINTS.md:

Single §1-C entry covering Stages 1+2A+3+4A-D + the

earlier §6-B + §7

§1.1 verdict amended from 🔴 OPERATOR-APPROVED DEVIATION

to ✅ PARITY (now genuinely byte/structure-faithful)

Sign-off block + commit trailer

### Resume after Stage 4E

Phase 2 step 14 — wire outbound_init() + metis_wire_bind() + ForceCandC::prime + Ep6RecvThread::start + Ep2SendThread::start into HL2Stream::session_open() in the correct temporal sequence.

### Wire status

Wire-INERT today.  All wire-layer components built but nothing in HL2Stream::session_open calls them yet.

## 2026-06-06 (Saturday MID-DAY — STAGE 3 CHECKPOINT — historical)

Branch: tx-rebuild HEAD = d88ddd6.  Build clean, lyra.exe relinked at 09:54:09.  Five commits shipped today + 6-agent comprehensive TX audit done.  Stages 1+2A+3 of the §1-C sweep DONE; Stages 2B+4 remain (bundled — §5.10 WSAEventSelect lives in _radionet fields per reference, so it lands with §1.1 revert).

### Today's commits (newest first)

d88ddd6 §1-C Stage 3 — OutboundRing wait-all via condition_variable

2b78d9e §1-C Stage 2A — Router class → struct + free functions

0e2a375 §1-C Stage 1 — small reverts (wb_enable, push timeout, diag counters)

50b713b §6-B nit — drop metis_write_frame null-guard

3bfd248 §6-B + §7 — TU-scope MetisFrame + ForceCandC populate

### What got fixed to match reference (Stages 1+2A+3)

§1.3: std::atomic<long> wb_enable → volatile long (matches network.h:160)

§6.10: OutboundRing producer push — drop bounded 5s try_acquire_for + push_timeouts_*; unbounded acquire() mirroring reference Inbound/obbuffs

§6.12: Ep2SendThread datagrams_sent_/send_errors_ diagnostic counters dropped (no reference counterpart)

§5.9: class Router → plain struct + free functions (xrouter/register_sink/set_control_word/set_call_count/router_instance) + file-scope g_routers[] array, mirroring reference router.c structure verbatim

§6.9/6.11/6.14: OutboundRing wait-all via std::condition_variable + bool flags + single mutex (was 4 binary_semaphores + polling) — mirrors reference WaitForMultipleObjects(2, hsendEventHandles, TRUE, INFINITE) atomic wait-all, no polling, fully interruptible

### Remaining — Stage 4 (THE BIG ONE)

§1.1 networking-infrastructure revert + §5.10 WSAEventSelect bundled.

§1.1 (signed 🔴 2026-06-04) excludes from RadioNet a list of fields that the reference puts INSIDE _radionet.  Under strict "do as reference, period" §1.1 itself is a §6-Q3/Q5- class candidate.  Stage 4 puts these fields back into RadioNet:

Buffer pointers: RxBuff, TxReadBufp, ReadBufp, OutBufp, outLRbufp, outIQbufp

Thread/sem/event handles: hReadThreadMain, hsendEventHandles[2], hobbuffsRun[2], etc.

Waitable timer + WSA event: liDueTime, hDataEvent, wsaProcessEvents

RX seq counters: cc_seq_no, cc_seq_err

Networking ports: p2_custom_port_base, base_outbound_port

Then Ep6RecvThread + Ep2SendThread + OutboundRing read from prn->... instead of their own members.  §5.10 (recv() → WSAEventSelect) folds in because hDataEvent + wsaProcessEvents are _radionet fields.

Scope: multi-hour, multi-commit.  Planned sub-stages:

4A: Add fields to RadioNet (additive, no behavior change)

4B: Ep6RecvThread refactor + WSAEventSelect mechanism

4C: Ep2SendThread refactor (reads buffers from RadioNet)

4D: OutboundRing — buffers + sync primitives move into

RadioNet; class may dissolve into free functions

4E: PARITY_CHECKPOINTS §1-C entry finalized + §1.1 amendment

### Resume after Stage 4 completes

Phase 2 step 14 — wire ForceCandC::prime + metis_wire_bind + WSAEventSelect setup into HL2Stream::session_open() in the correct temporal sequence.

### Wire status

Wire-INERT today.  All wire-layer components built but nothing in HL2Stream::session_open calls them yet.

## 2026-06-06 (Saturday MID-DAY — AUDIT PAUSE BOOKMARK — historical)

Branch: tx-rebuild HEAD = 50b713b.  Build clean, lyra.exe relinked at 09:27:22.  All §6-B + §7 work shipped + audited 20/20 PASS this morning.

### Done this morning (2 commits, ALL like reference)

3bfd248 §6-B parity correction sweep + §7 ForceCandC populate

Hoisted metis_write_frame to wire/MetisFrame.{h,cpp} TU-scope

TU-scope g_metis_out_seq_num (was Ep2SendThread::out_seq_num_)

TU-scope socket/dest globals (were Ep2SendThread members)

DROPPED Ep2SendThread::send_lock_ mutex (no reference counterpart)

§1.1 row split: RX seq stays / TX seq moves

ForceCandC::prime + prime_pass byte-for-byte from networkproto1.c:106-139

50b713b §6-B nit — dropped metis_write_frame null-guard (no reference

counterpart; audit-caught NOTE-D)

Two parallel audits run: parity (20/10 PASS) + rule-compliance (10/10 PASS)

### Pause reason

Operator-requested comprehensive TX audit of ALL work since the rebuild started (30 commits since 0348238 Phase 0 mapping doc 2026-06-04).  Sections to audit against the reference:

§1 RadioNet (2171fb9/dcbf9d1)

§2 RbpFilter / RbpFilter2 (228ffd8)

§3 HPSDR family + dispatch globals (f3ccc51/55ae658/782e65d)

§4a/§4b-1/§4b-2/§4c FrameComposer 19-case dispatch

(93e1511/5fc4192/4446916/f06a796)

§5 / §5-A Ep6RecvThread + Router (c8baa63/a6be425/b988177/6bbf449)

§6 / §6-A Ep2SendThread + OutboundRing (4c580a1/89ab298)

§6-B + §7 already audited this morning (skipping re-audit)

### Resume point after audit

Resume here: Phase 2 step 14 — wire-up `ForceCandC::prime` + `metis_wire_bind` into `HL2Stream::session_open()` in the correct temporal sequence (bind → prime → Ep6 start → Ep2 start).

### Wire status

Wire-INERT today.  Nothing in HL2Stream::session_open calls the new wire-layer components yet.  Step 14 (next commit after audit closes) flips the wire on.

## 2026-06-05 (Friday EOD)

Project: lyra-cpp TX Wire-Layer Rebuild (Task #112) — branch tx-rebuild

### Done today

§5 EP6 Receive Path — Ep6RecvThread + Router (xrouter/twist) populated (c8baa63)

§5-A 9-fix parity correction (a6be425) including 🔴 CRITICAL RX-audio bug (IQ unpack scale 48 dB too quiet — caught before any consumer wired)

§5-A doc completeness (b988177)

§6 EP2 Send Path — Ep2SendThread + OutboundRing populated (4c580a1)

§6-A 6-fix parity correction (89ab298) — MMCSS priority, per-family dispatch, CW bit-shift revert to verbatim, CW state per-sample reads, binary_semaphore UB guard, Rule 24 inline token

4 audit passes total (2 per §5 / §6) — all PASS after corrections

### Where we are

Branch tx-rebuild HEAD = 89ab298. §5 + §6 of the wire-layer rebuild are shipped + doubly-verified (parity-audit + rule-audit + correction sweeps + re-audits, all clean). Wire-layer is still wire-inert — no Phase 1 code path instantiates the new components.

### Next up

§7 — `ForceCandC` priming + ANY remaining wire-layer pieces. Reference: networkproto1.c::ForceCandCFrame (lines 134-139) + ForceCandCFrames (lines 106-132). Small commit — just the 3-frame priming burst that runs at session start before the EP2 send loop begins.

After §7 completes the §1c-listed wire-layer skeletons, the next phase is Phase 2 step 14 — wire-up: instantiate Ep6RecvThread + Ep2SendThread + OutboundRing + Router + ForceCandC + FrameComposer into HL2Stream's session-open path.

### Pending / parked

Task #114 TX-policy plumbing — ATT-on-TX, panadapter offset, PA-enable safety, plus the §6 FIXMEs: WriteMainLoop_generic for non-HL2, EER mode (§6.4), PeakFwdPower/PeakRevPower helpers (§5 telemetry cases 0x08/0x10)

No hardware bench yet — gateware-watchdog kill-test, real-antenna TX, foot-switch HW-PTT all parked until Phase 2 wire-up + verification

Don't push to GitHub (standing rule)

### Commit summary (today)

89ab298 fix(wire): §6-A parity correction sweep — 6 fixes
4c580a1 feat(wire): populate §6 Ep2SendThread + OutboundRing
b988177 docs(parity): §5-A Lyra-native additions table — 2 audit-flagged rows
a6be425 fix(wire): §5-A parity correction sweep — 9 fixes
c8baa63 feat(wire): populate §5 Ep6RecvThread + Router

### Standing rules in effect

Rule 24 "always verify against reference" — applied per commit, audit-verified

2026-06-05 directive "reference = make Lyra the same" — driving all Q-decisions

§6 EER mode + non-HL2 dispatch + Peak power helpers all deferred to Task #114 with FIXMEs

Operator hardware = HL2+/AK4951; no ANAN P1/P2 hardware available

*Session log started 2026-06-05.  Older entries (if any) below this line as they accumulate.*